EN

EN

EUROPEAN COMMISSION Brussels, 26.10.2022
SWD(2022) 545 final PART 3/4

COMMISSION STAFF WORKING DOCUMENT

IMPACT ASSESSMENT REPORT

Accompanying the document Proposal for a Directive of the European Parliament and of the Council

on ambient air quality and cleaner air for Europe (recast)

{COM(2022) 542 final} - {SEC(2022) 542 final} - {SWD(2022) 345 final} - {SWD(2022) 542 final}

148

TABLE OF CONTENTS TABLE OF CONTENTS ……………………………………………………………………………………………………………. 148 4. POPULATION EXPOSURE ………………………………………………………………………………………………. 151 5. HEALTH IMPACTS …………………………………………………………………………………………………………. 153 6. ECOSYSTEM AND OTHER NON-HEALTH IMPACTS ……………………………………………………… 158 7. MACRO-ECONOMIC IMPACTS ………………………………………………………………………………………. 163 ANNEX 6: POTENTIAL POLICY MEASURES (OR INTERVENTIONS) ……………………………………… 167 1. POLICY MEASURES IN THE DIFFERENT POLICY OPTIONS ………………………………………….. 167 2. ASSESSMENT OF POTENTIAL POLICY MEASURES AND THEIR IMPACTS ………………….. 168 2.1 Intervention area A: Regular review of EU air quality standards……………………………………. 170 2.2 Intervention area B: Type of EU air quality standards ………………………………………………….. 175 2.3 Intervention area C: Actions when exceedances occur …………………………………………………. 180 2.4 Intervention area D: Air Quality Plan Involvement ……………………………………………………… 186 2.6 Intervention area E: Enforcement tools ………………………………………………………………………. 188 2.7 Intervention area F: Information to the public …………………………………………………………….. 193 2.8 Intervention area G: Assessment regimes …………………………………………………………………… 197 2.9 Intervention area H: Number and typology of sampling points ……………………………………… 201 2.10 Intervention area I: Continuity of sampling points ……………………………………………………….. 205 2.11 Intervention area J: Siting of sampling points ……………………………………………………………… 208 2.12 Intervention area K: Data quality ………………………………………………………………………………. 212 2.13 Intervention area L: Additional pollutants ………………………………………………………………….. 216 2.14 Intervention area M: Transboundary air pollution ……………………………………………………….. 220 2.15 Intervention area N: Information in air quality plans ……………………………………………………. 222 2.16 Intervention area O: EU air quality standards for particulate matter (PM2.5) ……………………. 223 2.17 Intervention area P: EU air quality standards for particulate matter (PM10) …………………….. 228 2.18 Intervention area Q: EU air quality standards for nitrogen dioxide ………………………………… 232 2.19 Intervention area R: EU air quality standards for ozone ……………………………………………….. 237 2.20 Intervention area S: EU air quality standards for sulphur dioxide ………………………………….. 242 2.21 Intervention area T: EU air quality standards for carbon monoxide ……………………………….. 245 2.22 Intervention area U: EU air quality standards for benzene…………………………………………….. 247 2.23 Intervention area V: EU air quality standards for benzo(a)pyrene ………………………………….. 248 2.24 Intervention area W: EU air quality standards for lead …………………………………………………. 250 2.25 Intervention area X: EU air quality standards for arsenic ……………………………………………… 251 2.26 Intervention area Y: EU air quality standards for cadmium …………………………………………… 252 2.27 Intervention area Z: EU air quality standards for nickel ……………………………………………….. 253 2.28 Intervention area Ø: EU air quality standards for additional air pollutants ………………………. 254 3. POLICY OPTIONS …………………………………………………………………………………………………………… 255 3.1 Policy options to address environment / health shortcomings ………………………………………… 255 3.2 Policy options to address governance / enforcement shortcomings ………………………………… 256 3.3 Policy options to address air quality monitoring / assessment shortcomings ……………………. 259 3.4 Policy options to address air quality information shortcomings …………………………………….. 262 4. HOW DO THE OPTIONS COMPARE? ………………………………………………………………………………. 263

149

Maps for benzene – C6H6 Figure A5.19 - Benzene annual mean concentrations for baseline (Base) and MTFR for 2020, 2030 and 2050. Calculations are made on the EMEP 0.1o grid. For details (including on bias correction), please see the underpinning support study on the revision of the Ambient Air Quality Directives. Base 2020

Base 2030

MTFR 2030

Base 2050

MTFR 2050

150

Maps for benzo(a)pyrene - BaP Figure A5.20 - BaP annual mean concentrations for baseline (Base) and MTFR for 2020, 2030 and 2050. Calculations are made on the EMEP 0.1o grid. For details (including on bias correction), please see the underpinning support study on the revision of the Ambient Air Quality Directives. Base 2020

Base 2030

MTFR 2030

Base 2050

MTFR 2050

151

- POPULATION EXPOSURE In the next step of the analysis, the air pollutant concentrations have been translated into estimates for population exposure. The number of people exposed in the EU-27 above selected annual mean concentration ranges is presented for the pollutants PM2.5 and NO2. All Baseline, MTFR (or MFR) and optimised (OPT) scenarios are presented. The exposure calculations follow the same trends as seen for the station site calculations (results of which are included in the main SWD).
Figure A5.21– Number of people exposed above selected annual mean concentrations in the EU-27 for PM2.5 and NO2

As in the station calculations, the optimised scenarios do not attain their goals, with the exception of the less ambitious optimisations of 20 and 15 µg/m3 with less than 80 thousand

152

inhabitants exposed above 15 µg/m3. This is well within the uncertainty of the calculations. By 2050, all scenarios come close to attaining the WHO recommended NO2 concentration level of 10 µg/m3 but still with four to six million inhabitants exposed above this level. Population exposure and source contributions The results on population exposure are further broken down to show the split into the different sources of pollution at different levels of annual mean concentration.50 The following points can be noted: PM2.5
 530 000 inhabitants are exposed to PM2.5 concentrations higher than 25 µg/m3 in 2020 and this is chiefly attributable to residential emissions of primary PM2.5;  16 000 inhabitants are exposed to PM2.5 concentrations higher than 25 µg/m3 in 2030 but this number is well within the uncertainty of the methodology;  In 2030, remaining high annual mean concentrations are mainly caused by residential emissions;
 Where low annual mean concentrations prevail, the relative importance of secondary sources as well as natural sources increases;
 In 2030, non-exhaust PM2.5 emissions from road transport become a significant source contribution in some cities, notably Nordic countries. Non-exhaust emissions remain unchanged for all scenarios;  Local primary PM2.5 sources (i.e. all sources in lower case letters in figures below) that are emitted from within the ± 0.4o window, account for 22% of the total PM2.5 European exposure in 2020.

NO2  In 2020, the major source of NOX contributing to NO2 concentrations for all exposure levels is local road traffic, i.e. emitted from within the ± 0.4o window;  In 2030, road traffic contributes very little and the dominant source leading to exposures above 10 µg/m3 is shipping. There is a large degree of uncertainty in these local emission sources at ports.

BaP  In 2020, the dominant source for BaP is residential heating, followed by some individual industrial emissions;  In 2020, around 80 million inhabitants were exposed above the current EU limit value of 1 ng/m3. This was mostly in Poland and in Northern Italy. In 2030, this is reduced to 15 million;

50 The underlying support study contains graphics showing the population exposed over a range of annual mean concentration, as well as the relative source contributions for the given exposure level.

153

 In 2030, residential heating remains the dominant source at concentration levels of up to 2 ng/m3, with industrial sources dominating above 2 ng/m3;  With respect to industrial sources these exceedances are chiefly the result of one individual industrial plant in Northern Italy, Vicenza, and some lesser contributions in Spain and Poland. These emissions remain uncertain, both in present day and how they will evolve in the future.

The optimised calculations show similar results to the baseline scenario and MTFR scenarios but with a general reduction in concentrations from baseline scenario to MTFR.

- HEALTH IMPACTS
Results for attributable mortality (Tier 1) The impact of the various scenarios on the total number of yearly attributable deaths in the EU-27 for the three pollutants under consideration (PM2.5, NO2, O3) is shown in the bar graphs in Figure A5.22 (total number of premature deaths) and Figure A5.23 (relative differences between the baseline and the scenarios). These charts and numbers refer to the health impact above the WHO air quality guideline concentration levels; all excess mortality caused by concentrations below these cut-offs is not taken into account.
For particulate matter, the relative impact of the policy scenarios depends on the nature of the scenario. In case the baseline concentrations are already close to the target concentrations of the scenarios (as e.g. for the 20 μg/m3 scenario in 2030 and the 15 μg/m3 in 2050), the health impact of the scenarios is rather limited (15% for the 20 μg/m3 scenario in 2030 and 8% for the 15 μg/m3 scenario in 2050). For all other scenarios, the health impact for the OPT scenarios is significant (at least 38% in 2030 and at least 46% in 2050), and in many cases the difference between the health impact of the policy scenarios and the MTFR scenario is rather limited. As an example, the difference in health impact between the 10 μg/m3 on the one hand, and the MTFR on the other hand is only 8% in 2030 and only 4% in 2050.
For nitrogen dioxide, the impact (relative to the baseline) for the policy scenarios depends on the year under consideration. For 2030, the 20 μg/m3 has only a limited impact (2%), while the impact gradually increases for the more stringent scenarios (12%, 16%, and 20% respectively for 15 μg/m3, 10 μg/m3 and 5 μg/m3). In 2050, the impact (relative to the baseline) of the 15 μg/m3 scenario is small (1%), while the impact for the other policy scenarios is very similar to the impact of the MTFR scenario (14% reduction for both the 10 μg/m3 and 5 μg/m3, compared to 16% for MTFR). Finally, for ozone, the impact of the policy scenarios is small, with especially only marginal reductions for the 20 μg/m3 and the 15 μg/m3 scenario. The support studies in its annexes provides further geographical breakdown of the results reported here at aggregate EU-27 level, indicating a rather stable spatial pattern across scenarios.

154

Figure A5.22 – Number of yearly premature deaths in the EU-27 caused by the exposure to air pollution at levels above the WHO air quality guidelines for all scenarios for three pollutants (PM2.5, top-left, NO2, top-right, O3, bottom) based on the outcome of the modelling applied for this impact assessment51

51 Notes: Impacts for the four reporting years considered in the study (2015 in blue, 2020 in cyan, 2030 in orange and 2050 in green) are included. The filled bars and the numbers refer to the central estimate (rounded to the nearest 100 for NO2 and the nearest 1000 for PM2.5, respectively), while the black lines provide the 95-percentage uncertainty estimate based on the uncertainty on the relative risks.

155

Figure A5.23 – Relative impact of the scenario on the number of yearly premature deaths in the EU-27 caused by the exposure to air pollution at levels above the WHO air quality guidelines for three pollutants (PM2.5, top- left; NO2, top-right; O3, bottom) based on the outcome of the modelling applied for this impact assessment 52

Results for attributable morbidity (Tier 2 and Tier 3) Figure A5.24 provides an overview of the relative impact of the scenarios on the morbidity from the second (morbidity according to HRAPIE) and third tiers (additional health outcomes beyond HRAPIE: stroke, lung cancer and asthma in children). For all health outcomes, the results correspond qualitatively and quantitatively with those for the chronic mortality caused by PM2.5 exposure. The relative impact of the policy scenarios depends on the nature of the scenario. In case the baseline concentrations are already close to the target concentrations of the scenarios, the health impact of the scenarios is rather limited, in line with the results for mortality. For all other scenarios, the health impact for the policy scenarios is significant, and in many cases the difference between the health impact of the policy scenarios and the MTFR scenario is rather limited.

52 Notes: Impacts for the two future reporting years considered in the study (2030 in orange and 2050 in green)
are included.

156

Figure A5.24 – Relative impact of the scenarios on the morbidity in the EU-27 caused by the exposure to air pollution at levels above the WHO air quality guidelines for 2030 (top) and 2050 (bottom)53

Health impacts - summary results From this analysis, several conclusion can be drawn regarding the health impacts of the scenarios:

53 Notes: The various bars correspond to the various morbidity outcomes considered in the main analysis of the study (Tier 2 and Tier3).

157

 Under the baseline scenario, the mortality caused by exposure to PM2.5 and NO2 decreases significantly from 2015 to 2030. However, there would still be a considerable number of premature deaths each year observed in 2030, with tens of thousands of attributable deaths per year caused by the exposure to PM2.5 and thousands of deaths caused by the exposure to NO2: o For particulate matter, the baseline attributable mortality is larger in Eastern and Southern European countries, in comparison with the impact in most Northern and Western European countries (which is in line with the spatial pattern of the baseline emissions and the natural contribution). o The results for nitrogen dioxide reflect the nature of the nitrogen dioxide pollution: because concentration hotspots are mostly linked to important local shipping and traffic emissions, also the highest baseline mortality is observed at these locations.  The measures taken under the MTFR scenario have a significant impact on the health impact caused by the exposure to particulate matter (reductions with more than 55% in 2030, and with approximately 50% in 2050). Despite these strong reductions, a significant health impact remains under the application of the MTFR scenario, with more than 20 000 yearly attributable deaths in 2030 and more than 10 000 yearly attributable deaths in 2050. The impact of the MTFR scenario is somewhat more limited for the mortality caused by nitrogen dioxide pollution (relative reductions of 29% (2030) and 16% (2050) scenario).  The relative impact of the different policy scenarios depends on the nature of the scenario. In case the baseline concentrations are already close to the target concentrations of the scenarios (e.g. 20 μg/m3 scenario in 2030 and the 15 μg/m3 in 2050), the health impact of the scenarios is rather limited. For all other scenarios, the difference in health impact for the policy scenarios is similar to the health impact of the MTFR scenario: o For particulate matter, a strong regional difference in the impacts of the MTFR scenario is observed, as smaller relative impacts are observed in Southern Europe in comparison with other regions (due to the impact of the natural contribution and the minor reductions in shipping emissions).
o For nitrogen dioxide, the highest reduction in attributable mortality is observed at the hotspots for which the emissions are reduced by the greatest margin;  Results for morbidity show similar pattern to the results for mortality.

Economic valuation – benefits from reduced health impacts
In line with the valuation methods described in Annex 4, the costs of air pollution arising from impacts on human health and ecosystems were monetised. Comparing the costs estimated for the policy scenarios to the baseline scenario, yields a monetised estimate of the benefits associated with the reduced impacts observed in the policy scenarios. These benefits are summarised in chapter 6 of the main document, whereas Table A5.1 below presents the

158

underlying results per scenario for health impacts. The annexes to the support study contain further detailed breakdown of estimates per health outcome and pollutant. As can be seen from the results, there is a marked difference in the monetised human health benefits depending on the approach taken and on the scenario. Monetised benefits are smaller under the VOLY than VSL approach. The benefits increase, as expected, with the ambition of the scenario. The benefits reduce over time as more progress is made in the baseline, which erodes the additional benefit of further action under the mitigation scenarios.
Across all scenarios, mortality effects contribute the vast majority of the overall valued effects: the share of morbidity effects in the total valuation of human health benefits ranges from 1-6% across scenarios and years under the VSL approach, to 5-19% under the VOLY approach. Approach to valuing mortality Scenario 2020 2030 2050 VSL Baseline 739 444 332 VSL (PM2.5 at 20 µg/m3) - 408 - VSL I-3 (PM2.5 at 15 µg/m3) - 352 320 VSL I-2 (PM2.5 at 10 µg/m3) - 325 266 VSL I-1 (PM2.5 at 5 µg/m3) - 317 263 VSL MTFR - 303 256 VOLY Baseline 251 140 90 VOLY (PM2.5 at 20 µg/m3) - 128 - VOLY I-3 (PM2.5 at 15 µg/m3) - 109 87 VOLY I-2 (PM2.5 at 10 µg/m3) - 100 71 VOLY I-1 (PM2.5 at 5 µg/m3) - 97 70 VOLY MTFR - 92 68 Net VSL (PM2.5 at 20 µg/m3) - 36 - Net VSL I-3 (PM2.5 at 15 µg/m3) - 92 12 Net VSL I-2 (PM2.5 at 10 µg/m3) - 119 66 Net VSL I-1 (PM2.5 at 5 µg/m3) - 127 69 Net VSL MTFR - 141 77 Net VOLY (PM2.5 at 20 µg/m3) - 12 - Net VOLY I-3 (PM2.5 at 15 µg/m3) - 31 3 Net VOLY I-2 (PM2.5 at 10 µg/m3) - 40 19 Net VOLY I-1 (PM2.5 at 5 µg/m3) - 43 20 Net VOLY MTFR - 48 22 Table A5.1 – Costs and benefits (“Net” values) to society (valuation of health impacts – both mortality and morbidity, with approach to valuing mortality indicated in the first column) per year – central estimate (all values €bn 2015 prices, EU-27)

- ECOSYSTEM AND OTHER NON-HEALTH IMPACTS
Results for ecosystem areas exceeding critical loads for acidification and eutrophication
The following table presents the monetised impacts of air pollution on ecosystems (incl.  productive ones) as well as material damage (mainly built environment) under the baseline, policy and MTFR scenarios. The size of the damage in the baseline and scenarios reduces over time alongside further emissions reductions delivered through current policy. The monetised benefits increase with the ambition under each scenario, as further reduction in air

159

pollutant emissions are delivered. The scenarios and MTFR can deliver substantial benefits for ecosystems and from reduced material damage, however the aggregate size of these benefits is still smaller than the human health benefits. This section presents indicators on ecosystem impacts in terms of acidification and eutrophication from excess deposition of nitrogen (for acidification and eutrophication) and sulphur (for acidification). Results are calculated with the GAINS model using critical loads approved by the Air Convention in 2017.
Maps of ecosystem areas exceeding critical loads for acidification and eutrophication (percentages of area above critical load) from deposition of nitrogen and sulphur are shown in Figures A5.25 and A5.26 for the Baseline and MTFR scenarios and the years 2020, 2030 and 2050. Eutrophication is still a widespread problem in Europe, with an estimated 74% of all ecosystem areas exceeding critical loads. Despite improvements in 2030 and even further in 2050, still ~65% of ecosystem areas are expected to exceed critical loads for eutrophication in 2050 under the baseline scenario. Under the MTFR scenario, this is reduced to 48% in 2050. Acidification is much less of an issue, with 4.8% of ecosystem areas currently exceeding the critical loads, decreasing to 3.1% in 2030 and 2.4% in 2050 under the Baseline scenario (1.2% in 2050 under the MTFR scenario). The support study includes tables that differentiate the impacts of the different scenarios in terms of area shares of different types of ecosystems where critical loads for eutrophication and acidification are exceeded by Member State.

160

Figure A5.25 – Shares of ecosystem area exceeding critical loads for eutrophication
Baseline 2020

Baseline 2030

MTFR 2030

Baseline 2050

MTFR 2050

161

Figure A5.26 – Shares of ecosystem area exceeding critical loads for acidification Baseline 2020

Baseline 2050

MTFR 2050

Baseline 2050

MTFR 2050

Economic valuation – benefits from reduced material damage and impacts on ecosystems Table A5.2 presents the monetised impacts of air pollution on crops, productive forests and other ecosystems as well as material damage (mainly built environment) under the baseline, policy and MTFR scenarios. The size of the damage in the baseline and scenarios reduces

162

over time alongside further emissions reductions delivered through current policy. The monetised benefits increase with the ambition under each scenario, as further reduction in air pollutant emissions are delivered. The scenarios and MTFR can deliver substantial benefits from reduced damage, however the absolute size of these benefits is several orders of magnitude smaller than the human health benefits. Table A5.2 – Monetised material, crop and forest damage impacts per year. Gross values (upper part) and benefit from reduced damage relative to baseline (lower, NET part of table) – EUR million 2015 prices

2020 2030 2050 2020 2030 2050 2020 2030 2050

Material damage Crop damage Forest damage (LOW)
Baseline 1,136 662 442 10,691 9,877 9,459 19,050 17,975 17,374 (PM2.5 at 20 µg/m3)

633

9,809

17,906

I-3 (PM2.5 at 15 µg/m3)

481 430

9,689 9,415 17,752 17,321 I-2 (PM2.5 at 10 µg/m3

466 286

9,623 9,200 17,688 17,082 I-1 (PM2.5 at 5 µg/m3)

458 281

9,600 9,201 17,659 17,080 MTFR

436 269

9,472 9,110 17,486 16,954 NET at 20 µg/m3)

29

67

69

NET at 15 µg/m3)

181 12

188 44

222 52 NET at 10 µg/m3)

196 156

254 259

287 292 NET at 5 µg/m3)

204 160

276 258

316 293 NET MTFR

226 172

404 348

488 420

Ecosystem damage (LOW) Ecosystem damage (HIGH) Forest (HIGH) 54 Baseline 3,901 3,588 3,375 11,702 10,765 10,124

42,217 (PM2.5 at 20 µg/m3)

3,488

10,463

I-3 (PM2.5 at 15 µg/m3)

3,140 3,291

9,420 9,874

42,090 I-2 (PM2.5 at 10 µg/m3

2,883 2,585

8,648 7,754

41,505 I-1 (PM2.5 at 5 µg/m3)

2,726 2,443

8,177 7,330

41,501 MTFR

2,588 2,328

7,765 6,984

41,194 NET at 20 µg/m3)

101

302

NET at 15 µg/m3)

448 83

1,345 250

127 NET at 10 µg/m3)

706 790

2,117 2,370

712 NET at 5 µg/m3)

863 931

2,588 2,794

716 NET MTFR

1,000 1,047

3,000 3,140

1,023

54 Note that there is no difference between HIGH and LOW estimate for forest damage in 2020 and 2030 as only after 2030 different assumptions are used to monetise the reduced carbon sequestration potential due to forest damage.

163

- MACRO-ECONOMIC IMPACTS Air pollution has detrimental welfare impacts by affecting health outcomes. In addition, related healthcare expenditures, crop yield losses in particular due to ozone, absence from work due to illness (including of dependent children) and lower productivity at work (presenteeism) can imply a drag on the economy. Improving air quality can therefore bring economic gains. However, air pollution control comes at a gross cost, as it requires costly investments and purchases of abatement equipment. A priori, it is unclear whether air pollution control policies therefore lead to net economic gains or losses, and how these are distributed across stakeholders. To shed some light on these trade-offs, a macro-economic benefit-cost analysis was conducted by linking the GAINS model with the JRC-GEM-E3 model. This has been done in previous work, such as the First and Second Clean Air Outlook, and both models feature in a broader modelling toolbox e.g. in the assessment of the EU long-term climate strategy55. The key information that flows from GAINS to JRC-GEM-E3 is the abatement cost associated to further air pollution controls induced by more ambitious policy measures and targets. These costs serve as inputs into the JRC-GEM-E3 analysis (Figure A5.13).

Figure A5.13 – Air pollution mitigation or adjustment costs (EU total) beyond the baseline, for different policy scenarios. Source: GAINS model, IIASA (support study).

The JRC-GEM-E3 model represents the whole economy and the interactions between key actors: firms, households and governments in the EU and in the rest of the world. End-of-pipe abatement costs from GAINS are treated as costly (intermediate) expenditures on abatement goods and services, and therefore generate additional demand for the sectors that deliver these goods and services. Furthermore, the model captures the potential loss in competitiveness of firms that need to incur abatement costs by reflecting price-driven international trade flows. For households, a loss of income or raised expenditure on abatement technologies implies that less means are available to purchase other goods. The

55 Weitzel, M., et al. (2019). Model-based assessments for long-term climate strategies. Nature Climate Change, 9(5), 345-347. 0 2 4 6 8 20µg 15µg 10µg 5µg 15µg 10µg 5µg Billion € per year Crops Livestock Power sector Fossil fuels Industry Services Households 2030 2050

164

economic modelling framework covers these interactions to provide an economy-wide picture of the implications of additional air pollution control costs. On the benefit side, this analysis concentrates on productivity gains from clean air. The empirical basis stems from recent OECD work56 that quantifies the causal impact of PM2.5 pollution on productivity in the EU for the period 2000-2015. More specifically, labour productivity gains are derived by combining the point estimate on the impact of PM2.5 on GDP per worker, with the changes (compared to the baseline) in population-weighted PM2.5 concentrations from the GAINS model. The corresponding changes in labour productivity are fed into the JRC-GEM-E3 model, where labour constitutes an input the production process of the various economic sectors. The results are displayed in Table A5.3. The key insight is that all scenarios improve aggregate economic outcomes in the EU compared to a situation of unchanged policy. The most ambitious 5 μg/m3 scenarios imply larger gross costs, but these are more than compensated by productivity gains, as reflected by the positive impact on GDP and private consumption. With the exception of livestock-based agriculture, all sectors displayed raise output compared to the reference when productivity gains of clean air are accounted for. Table A5.3 – Economic outcomes of clean air policy in the EU – expressed as percentage change relative to reference baseline. Note that the first number in a cell represents the effect of gross costs only. The second number (after the vertical line) represents the net effect, i.e. benefits minus costs.57 >>> Option analysed >>>

W/ current standards Option I-1 (PM2.5 at
5 µg/m3) Option I-2 (PM2.5 at
10 µg/m3) Option I-3 (PM2.5 at
15 µg/m3) Sub-option I-1a Sub-option I-2a Sub-option I-3a Cost only | Net effect (benefit-cost) % change relative to reference 2030 2030 2030 2030 2050 2050 2050 Gross Domestic Product 0.00 | 0.10 -0.05 | 0.44 -0.04 | 0.38 -0.02 | 0.26 -0.03 | 0.36 -0.02 | 0.29 0.00 | 0.03 Private Consumption 0.00 | 0.12 -0.04 | 0.57 -0.03 | 0.49 -0.02 | 0.34 -0.02 | 0.46 -0.02 | 0.37 0.00 | 0.04

Sector output

Crops -0.02 | 0.15 -0.32 | 0.50 -0.26 | 0.45 -0.19 | 0.30 -0.30 | 0.36 -0.17 | 0.38 0.00 | 0.06 Livestock -0.09 | 0.05 -1.01 | -0.36 -0.62 | -0.05 -0.45 | -0.06 -0.91 | -0.37 -0.54 | -0.10 -0.01 | 0.05 Power sector 0.00 | 0.11 0.01 | 0.50 0.01 | 0.44 0.00 | 0.30 0.02 | 0.41 0.02 | 0.34 0.00 | 0.04 Fossil fuels -0.01 | 0.08 -0.11 | 0.32 -0.10 | 0.28 -0.09 | 0.18 -0.03 | 0.29 -0.04 | 0.24 0.00 | 0.03 Industry 0.00 | 0.13 0.02 | 0.63 0.01 | 0.53 0.02 | 0.38 0.01 | 0.51 0.00 | 0.40 0.00 | 0.05 Services 0.00 | 0.09 0.00 | 0.45 0.00 | 0.38 0.0 | 0.26 0.00 | 0.37 0.00 | 0.29 0.00 | 0.03

A few caveats are important to take into consideration when interpreting these results. Here, we focus exclusively on productivity benefits from clean air. This implies that other ‘market’ benefits are not included, such as reduced healthcare expenditures and increased crop yields. Furthermore, additional ‘non-market’ benefits, such as ecosystem impacts and reductions in premature mortality or life years lost due to air pollution, are not included in the results displayed in the table below. While these benefits are not included in the economy-wide assessment in this section, they are discussed in other sections of this report. The JRC-GEM- E3 modelling results furthermore include outcomes on employment changes by sector. In these simulations, it was assumed that wage setting is flexible such that it can fully

56 Dechezleprêtre, A., Rivers, N., & Stadler, B. (2019). The economic cost of air pollution: Evidence from Europe. OECD Economics Department Working Papers. 57 Based on general equilibrium modelling with the JRC-GEM-E3 model.

165

accommodate labour market adjustments. This implies that aggregate, national unemployment levels are driven by fundamental factors that are unaffected by clean air policy. In other words, this assumption implies that the results will not pick up any potential aggregate net job creation associated with increased GDP and output levels as shown, and the results may therefore be interpreted as conservative estimates. The results displayed in Table A5.4 indicate two consistent findings across all scenarios and years. First, we observe a creation of jobs in industry, which relates directly to the production of equipment required to abate emissions and the associated investments. While industry also faces increased abatement costs, in terms of net effect on jobs this is more than offset by increased demand for abatement goods from all sectors (including households). Second, the agricultural sector experiences job losses compared to the reference, which relates to output losses (livestock sector) or a transition of workers into industry (crops sector). Overall, the magnitude of the employment changes is limited in relative terms such that they may be largely absorbed by ongoing labour market dynamics (entry into and exit from the labour market). One caveat worthwhile mentioning here is that the productivity benefits are applied uniformly across all sectors. A stronger empirical evidence base would help refining (the sector-specific elements of) the analysis, e.g. by differentiating productivity impacts of air pollution for vulnerable workers. Table A5.4 – Employment transition across sectors in the EU. Source: JRC-GEM-E3. Given the assumption of flexible wage setting, positive and negative employment effects balance out for a given year and scenario. Adding of numbers in a given column of this table does not yield zero in all cases due to rounding. >>> Option analysed >>>

(PM2.5 at
20 µg/m3) Option I-1 (PM2.5 at
5 µg/m3) Option I-2 (PM2.5 at
10 µg/m3) Option I-3 (PM2.5 at
15 µg/m3) Sub-option I-1a Sub-option I-2a Sub-option I-3a Cost only | Net effect (benefit-cost) 1000 jobs, change rel. to reference 2030 2030 2030 2030 2050 2050 2050 Employment

Crops -1 | -2 -19 | -19 -18 | -18 -15 | -17 -17 | -16 -10 | -9 0 | 0 Livestock -2 | -3 -25 | -31 -23 | -29 -20 | -24 -19 | -22 -17 | -20 0 | -1 Power sector 0 | -2 0 | -8 0 | -7 0 | -6 0 | -5 0 | -4 0 | -1 Fossil fuels 0 | 0 0 | -1 0 | 0 0 | 0 0 | 0 0 | 0 0 | 0 Industry 3 | 24 34 | 115 30 | 104 25 | 81 23 | 81 15 | 66 0 | 7 Services 1 | -17 10 | -57 11 | -49 10 | -33 13 | -38 12 | -33 0 | -6

- SENSITIVITY ANALYSIS FOR IED IMPLEMENTATION
In order to quantify to the extent possible the impact of the proposal for a revised IED on air quality, a sensitivity analysis was conducted. Since it is not possible to precisely project the speed and scope of the IED implementation58, as this depends on the development and uptake of BAT across economic sectors, a series of assumptions was taken:

 For all non-agriculture sectors covered by the proposal for a revised IED, a 20% reduction in PM2.5, SO2, and NOx emissions in 2030 (additional to the reduction already

58 See explanations in the Impact Assessment accompanying the proposal for a revised Industrial Emission Directive (SWD COM(2022)111)

166

foreseen in the baseline), reflecting the likely best-case scenario of the IED implementation and based on assumptions presented in the impact assessment underpinning the revision of the IED.
 For agriculture sectors covered by the proposal for a revised IED, country-specific and livestock category-specific NH3 reduction rates estimated with the GAINS model for the third Clean Air Outlook, assuming an entry into force of the proposal in 2027. Typically, this results in about 1% to 4% reduction of national NH3 emissions (about 2% for the EU27 as a whole) beyond the baseline used in this impact assessment.

The modelling approach followed the following steps:

 projection of the impact on country specific industrial emissions of PM2.5, SO2, NOx for the year 2030;  projection of the impact on country specific NH3 emissions from agriculture for the year 2030;
 these newly estimated emissions of all pollutants were used in the EMEP model simulations for 2030 to estimate PM2.5 and NO2 concentrations and station compliance and exposure.

The following table present the change in station concentrations from the 2030 Baseline to the 2030 IED sensitivity.

Average change in mean concentration levels at sampling points
Number of sampling points that fall into / drop out of the respective concentration bands under IED sensitivity assumptions
PM2.5 Mean absolute change (µg/m3) Mean relative change (%) 0 – 5 (µg/m3) 5 – 10 (µg/m3) 10 – 15 (µg/m3) 15 – 20 (µg/m3) 20 - 25 (µg/m3) > 25 (µg/m3)

-0.13 -1.6 35 -29 -6 0 0 0 NO2 Mean absolute change (µg/m3) Mean relative change (%) 0 – 10 (µg/m3) 10 – 20 (µg/m3) 20 – 30 (µg/m3) 30 – 40 (µg/m3) > 40 (µg/m3)

-0.09 -0.89 17 -15 -2 0 0

It should be noted that, due to the methodological limitations of any sensitivity analysis, these results cannot be used for drawing exact conclusion on impacts. They do however provide a useful indication of the impact of the revised IED implementation on pollution concentration, and show that these impacts are likely to be very small compared to the baseline used in the core of this impact. For more information, see the underpinning support study.

167

ANNEX 6: POTENTIAL POLICY MEASURES (OR INTERVENTIONS)
1. POLICY MEASURES IN THE DIFFERENT POLICY OPTIONS This impact assessment considers a total of 69 potential specific policy measures - these measures are based on WHO recommendations (including as published in 2021), as well as stakeholder feedback to the Inception Impact Assessment and preliminary expert consultations (with those responsible for air quality monitoring, modelling and planning). Table A6.1 – Overview of all 69 specific measures considered in this impact assessment

Focus on AQ legislative framework 35 J1 Revise macro-scale siting of sampling points 1 - Merge provision of Directives 2008/50 and 2004/107 36 J2 Revise micro-scale siting of sampling points 2 A1 Introduce review triggered by scientific progress 37 J3 Introduce obligation for spatial representativeness 3 A2 Introduce review triggered by technical progress 38 K1 Revise AQ monitoring data quality objectives
4 A3 Introduce option to notify stricter standards 39 K2 Introduce up-to-date data at all sampling points 5 A4 Introduce a list of priority pollutants
40 K3 Introduce AQ modelling data quality objectives 6 B1 Introduce additional short-term standards
41 K4 Revise approach to AQ assessment uncertainty 7 B2 Introduce additional alert/information thresholds
42 L1 Introduce concept of monitoring at ‘super-sites’ 8 B3 Revise definition of average exposure standards 43 L2 Introduce obligations to monitor more pollutants 9 B4 Introduce guidance on addressing exceedances 44 L3 Revise list of VOC to monitor 10 B5 Introduce limit values for additional air pollutants 45 M1 Introduce methodology to assess transboundary 11 C1 Revise obligations triggered by exceedances
46 M2 Revise obligations for transboundary cooperation 12 C2 Revise/clarify definition of ‘as short as possible’ 47 N1 Revise the information in air quality plans
13 C3 Revise short-term action plans & air quality plans

Focus on EU air quality standards 14 C4 Introduce additional short-term action plans 48 O1 Revise standards for annual PM2.5 15 C5 Introduce requirement to update air quality plans 49 O2 Introduce standards for daily PM2.5 16 D1 Revise requirements to involve stakeholders
50 O3 Revise average exposure standards for PM2.5 17 D2 Introduce a ‘one zone, one plan’ requirement 51 P1 Revise standards for annual PM10 18 E1 Introduce minimum levels for financial penalties
52 P2 Revise standards for daily PM10 19 E2 Introduce right to health damage compensation
53 P3 Introduce average exposure standards for PM10 20 E3 Introduce a fund to be fed by penalties paid 54 Q1 Revise standards for annual NO2 21 E4 Introduce an explicit ‘access to justice’ clause 55 Q2 Revise/introduce standards for hourly/daily NO2 22 F1 Revise provisions related to up-to-date data 56 Q3 Introduce average exposure standards for NO2 23 F2 Introduce requirement to provide AQ health data 57 R1 Introduce standards for peak-season O3 24 F3 Introduce use of specific communication channels 58 R2 Revise standards for 8-hour O3 25 F4 Introduce requirements for harmonised AQ index 59 R3 Introduce average exposure standards for O3

Focus on AQ monitoring, modelling, plans 60 S1 Revise standards for annual SO2 26 G1 Revise rules related to indicative sampling points 61 S2 Revise standards for daily/hourly SO2 27 G2 Introduce requirements for AQ modelling
62 T1 Revise standards for daily/8-hour CO 28 G3 Revise rules for regular review of AQ assessment 63 U1 Revise standards for annual benzene 29 H1 Revise minimum number of sampling points 64 V1 Revise standards for annual benzo(a)pyrene 30 H2 Simplify combined PM10/PM2.5 monitoring 65 W1 Revise standards for annual lead 31 H3 Simplify the definitions of sampling points types 66 X1 Revise standards for annual arsenic 32 I1 Introduce obligations to maintain sampling points 67 Y1 Revise standards for annual cadmium 33 I2 Introduce obligations to monitor long-term trends 68 Z1 Revise standards for annual nickel
34 I3 Introduce a protocol for relocated sampling points 69 Ø1 Introduce standards for additional air pollutants

168

- ASSESSMENT OF POTENTIAL POLICY MEASURES AND THEIR IMPACTS Assessment criteria and indicators Broad impact category Indicator
Indicator # Environmental impacts

(including air pollutant concentrations)

Concentration levels of air pollutants, at (a) background locations, and (b) ‘hot-spot’ (incl. both traffic and industry-related) locations, and their development over time; #1 Health impacts of air pollution, for example the health impacts resulting from exposure to particulate matter (PM2.5 and/or PM10), nitrogen dioxide and ozone; #2 Ecosystem impacts of air pollution, including acidification, eutrophication, ozone damage to vegetation and agricultural yields; #3 Links between air pollution and climate change, including increased ozone levels due to global warming, and co-benefits or trade-offs between climate and air pollution abatement measures; #4 Economic impacts Cost to society due to air pollution, including health and healthcare impacts and costs, lost working days, crop and animal value loss, losses to other assets and other costs avoided by taking action to reduce air pollution; #5 Measures needed to meet EU air quality standards - and their costs, including costs for key economic sectors, and regional differences across the EU of the costs and benefits of the air pollution abatement measures; #6 Positive and negative impacts on the EU’s international competitiveness, including tapping into innovation potential for clean air technologies; #7 Social impacts Effects of air pollution on sensitive population groups, including children, pregnant women, elderly citizens and those suffering from pre-existing conditions; #8 Societal impacts of air pollution and societal impacts of air pollution abatement measures, incl. resulting inequalities (i.e. who is most affected, who bears costs); #9 Effects of measures to address air pollution on employment; #10 Synergies Synergies with other goals of the (upcoming) EU Zero Pollution Action Plan on air, water and soil. This includes premature death reduction (indicator 2) and ecosystem impact (indicator 3) goals. It additionally reflects the synergic role of indoor air pollution (notably in terms of exposure and health impacts) or co-benefits in reducing noise pollution. #11 Administrative burden Administrative burden of air quality management, in particular as relates to air quality assessment regimes (including monitoring, modelling, and reporting of related data) – for a detailed assessment see Annex 3. #12

Each of the above indicator is scored for each specific potential policy measure in a qualitative manner, taking into account the quantitative assessment provided in Annex 3 and Annex 5 where possible. Note that for several indicators no extensive quantification has been possible, due to the lack of available data per specific potential policy measure. In these cases the assessment is based on expert judgement provided via the underpinning support study.

Score Description +++ Very significant direct positive impact or benefit ++ Significant direct positive impact or benefit + Small direct positive impact or benefit (+) Indirect positive impact or benefit +/- Both direct positive and negative impacts, and balance depends on how implemented
0 No impact or only very indirect impacts (-) Indirect negative impact or cost - Small direct negative impact or cost - - Significant direct negative impact or cost - - - Very significant direct negative impact or cost High Benefits significantly outweigh costs of measure Medium Benefits on balance outweigh costs of measure Low Benefits close to or even below costs of measure High (*) Potential high benefits, but significant questions as to whether the measure can deliver outcome

169

For a summary overview the scoring of the twelve indicators for each specific potential policy measure is presented via an overview table, as per the following logic. Note that indicator #6 features twice, i.e. under economic impact and under cost.

Policy Measure Env. impact Soc. impact Eco. impact Cost Policy synergy Benefit to cost Included in policy options Code Short-hand description of policy measure #1 #2 #3 #4 #8 #9 #10 #5 #6 #7 (#6)

#12 #11 High Medium Low High (*) Reference to which policy options and/or sub-options include this measure

See above for indicator code,
see legend below for scoring See
legend

Note that indicator #12 on the administrative burden is based on the estimates presented in Annex 3. Combined additional annualised one-off and recurring administrative burden in the category of less than 10 000 Euro per year are marked ‘(-)’, between 10 000 and 100 000 Euro as ‘-‘, between 100 000 and 1 000 000 Euro as ‘–‘, and above 1 000 000 Euro as ‘—‘. Stakeholder views
A targeted stakeholder survey asked for views on each potential specific policy intervention (see Annex 2 for details), and consulted with public authorities, civil society & NGOs, industry & businesses, and research & academia. The number of responses differed by policy are consulted upon.
For intervention areas A through N, in total 93 replies were received from:  Public authorities (43);  Civil society & NGOs (12);  Industry & businesses (14);  Research & academia (22). For intervention areas O to Ø, in total 139 replies were received from:  Public authorities (53);  Civil society & NGOs (12);  Industry & businesses (26);  Research & academia (42). Stakeholder views per potential specific policy intervention are summarised below.

170

2.1 Intervention area A: Regular review of EU air quality standards
A1 Introduce review of EU air quality standards triggered by scientific progress

Policy Measure Env. impact Soc. impact Eco. impact Cost Policy synergy Benefit to cost Included in policy options A1 Introduce review triggered by scientific progress ++ (+) (+) + (+) (+) 0

0 0 0 0

(-) + High

Policy option I-6
Focus of measure: Introduce a mechanism for adjusting EU air quality standards upon publication of new scientific advice (including, but not limited to, the publication of new WHO Air Quality Guidelines). Description of measure: Appropriate mechanisms are needed to flexibly adapt to evolving science to protect human health. Article 32 of Directive 2008/50/EC and Article 8 of Directive 2004/107/EC provided grounds for once-off reviews on the basis of specific evidence (e.g. WHO Air Quality Guidelines or reduction potentials in Member States), but do not provide a mandate or obligation for regular reviews. Three possible intervention variants exist under this intervention to ensure that Ambient Air Quality Directives reflect latest scientific advice:
1. Introduce a binding schedule of reviews of scientific progress to be undertaken by the Commission - under this variant the Commission would undertake a periodic review of scientific progress related to air pollutants.
2. Introduce a mechanism for adjusting EU air quality standards upon publication of new WHO Air Quality Guidelines - under this variant the Commission would undertake a WHO Air Quality Guidelines related review of scientific progress related to air pollutants, with a view to presenting a proposal to amend the Directives to the European Parliament and the Council.
3. Introduce a mechanism for adjusting air quality standards based on (other) latest scientific advice - under this variant the Commission would undertake a review of new scientific knowledge of related to air pollutants, with a view to presenting a proposal to amend the Directives to the European Parliament and the Council. Stakeholder views: A targeted survey provided feedback on ‘to which extent would this specific intervention address the identified (related) shortcomings’.

171

Summary: Introduction of a mechanism that will provide a basis for the alignment of the Ambient Air Quality Directives with the latest scientific knowledge will directly contribute towards reductions in air quality concentrations. Meeting the direct objective of air quality reduction will subsequently indirectly protect EU population from harmful exposure to air pollution and indirectly benefit ecosystems. Direct costs estimated for this intervention are small administrative costs for the Commission.

A2 Introduce review of EU air quality standards triggered by technical progress

Policy Measure Env. impact Soc. impact Eco. impact Cost Policy synergy Benefit to cost Included in policy options A2 Introduce review triggered by technical progress (+) (+) (+) (+) 0 0 0 0 0 0 0

(-) (+) Low

Sub-option I-6a
Focus of measure: Introduce a mechanism for adjusting EU air quality standards based on technical progress in air pollution reduction. Description of measure: This intervention would introduce a mechanism for adjusting EU air quality standards based on technical progress in air pollution reduction. Accordingly, the Commission would undertake regular reviews of technical progress related to abatement techniques for air quality pollutants and the cost of implementing standards that are more stringent. Stakeholder views: A targeted survey provided feedback on ‘to which extent would this specific intervention address the identified (related) shortcomings’.

Summary: This measure introduces a mechanism for adjusting EU air quality standards based on technical progress in air pollution reduction. This intervention would formalise consideration of technological progress in the Ambient Air Quality Directive and could have a small positive indirect impact on improvements in air quality concentrations as advances in the technological knowledge might lead to revisions in the Ambient Air Quality Directive due to the enhanced technical feasibility of its implementation. However, the process would be driven by technology considerations, not health considerations, and therefore addresses the

172

objective of protecting human health only to some extent. Direct costs estimated for this intervention are small administrative costs for the Commission.

A3 Introduce option to notify stricter standards by Member States

Policy Measure Env. impact Soc. impact Eco. impact Cost Policy synergy Benefit to cost Included in policy options A3 Introduce option to notify stricter standards (+) (+) (+) (+) (+) 0 0 0 0 0 0

(-) 0 High

Policy option I-6 Focus of measure: Introduce a provision in the Ambient Air Quality Directives to allow for EU Member States to adopt more stringent standards in light of the new technical and scientific progress coupled with an obligation to notify the Commission. Description of measure: The European Commission would introduce a requirement to ensure that EU Member States notify the Commission if they adopt more stringent standards within their jurisdiction in light of the new technical and scientific progress. EU Member States already have the possibility to adopt more stringent protective measures in accordance with Article 193 TFEU which must be notified to the Commission. The intervention would explicitly enshrine this possibility with regard to stricter air quality standards in secondary legislation and elaborate on the obligation to notify the European Commission with a view to collecting information on technical and scientific knowledge and national/local standards surpassing the EU standard and enabling information sharing across Member States. Stakeholder views: A targeted survey provided feedback on ‘to which extent would this specific intervention address the identified (related) shortcomings’.

Summary: This measure introduces a provision to allow EU Member States to adopt more stringent standards in light of the new technical and scientific progress coupled with an obligation to notify the Commission. This intervention has a potential to have a small indirect impact on reducing air pollution concentrations as it would contribute to sharing of information, including on scientific and technical data that can be used by the EU and other EU Member States. This intervention has been assessed under the assumption that it will enhance the Commission’s evidence base regarding Member State policy action at EU level.

173

Direct costs estimated for this intervention are small administrative costs for EU Member State competent authorities. The benefit cost ratio of this measure is considered high as low administrative burden would lead to an improved knowledge base.

A4 Introduce a list of priority air pollutants

Policy Measure Env. impact Soc. impact Eco. impact Cost Policy synergy Benefit to cost Included in policy options A4 Introduce a list of priority pollutants
(+) (+) (+) (+) 0 0 0 0 0 0 0

- 0 Low

Sub-option I-6b Focus of measure: Keep and periodically update a list of priority air pollutants with a view to monitoring air pollutants of emerging concern. Description of measure: Directives 2004/107/EC and 2008/50/EC establish standards and objectives for a number of air pollutants, namely PM2.5, PM10, SO2, NO2, NOx, Pb, CO, C6H6, O3, As, Cd, Ni and polycyclic aromatic hydrocarbons, to protect human health and the environment. This intervention would mandate the Commission to establish and periodically update a list of additional priority air pollutants with a view to monitoring air pollutants of emerging concern. Accordingly, the Commission would regularly update a “watch list” for emerging substances as part of the latest technical and scientific review and to demand their monitoring at Member State level. This measure would provide a first step for improving knowledge of and developing standards for air quality pollutants that are currently not covered in the Ambient Air Quality Directives. The Commission would be responsible for the watch list, but Member States would carry out the monitoring. Stakeholder views: A targeted survey provided feedback on ‘to which extent would this specific intervention address the identified (related) shortcomings’.

Summary: This measure aims to keep and periodically update a list of priority air pollutants with a view to monitoring air pollutants of emerging concern. This intervention is likely to have a small indirect impact on air quality as the monitoring of identified priority pollutants could eventually lead to regulating them in the Ambient Air Quality Directive. Black carbon (BC), Ultrafine particles (UFP), ammonia (NH3), PFAS (Per- and polyfluoroalkyl

174

substances), dioxins and PCB (polychlorinated biphenyl) have been specifically highlighted as possible priority air pollutants. Direct costs estimated for this intervention are small administrative costs for the Commission. Additional burden would potentially be borne by the EU Member States if they were required (or voluntarily choose) to monitor priority emerging air pollutants, in particular if the content of the list changed frequently.

175

2.2 Intervention area B: Type of EU air quality standards
B1 Introduce additional short-term standards

Policy Measure Env. impact Soc. impact Eco. impact Cost Policy synergy Benefit to cost Included in policy options B1 Introduce additional short- term standards
+ 0 0 0 0 0 0 0 0 0 0

(-) 0 High

Policy option II-2 Focus of measure: Establish short-term EU air quality standards (daily or hourly) for additional air pollutants that currently only have annual or seasonal standards e.g. PM2.5. Description of measure: The Ambient Air Quality Directives sets short-term standards only for certain pollutants. There are cases where the WHO Air Quality Guidelines provide a recommendation for short-term exposure levels for additional pollutants. For example, for PM2.5 there is an EU annual limit value, but no 24-hour standard; for SO2 there are EU standards for 1-hour and 24-hour periods, but no 10-minute standard; for NO2 there is an EU standard for 1-hour exposure, but no 24-hour standard. This intervention explores the regulatory change needed to underpin the formulation of additional short-term standards for various pollutants for which currently only long-term standards (annual-mean) exist, or alternative short-term averaging periods, to achieve greater alignment with the latest WHO Air Quality Guidelines. Stakeholder views: A targeted survey provided feedback on ‘to which extent would this specific intervention address the identified (related) shortcomings’.

Summary: This is a facilitating measure. It goes hand-in-hand with (and the true impacts are determined by) the ambition of the standards set under other interventions (O2, P2, Q2, R2, S2 and T1). This intervention provides the facilitating legal basis for such standards to be set, and hence is an important component of a wider solution that could be effective in improving air quality and thereby improving health protection. As such this measure has only low direct costs, but the potential for high benefits. Stakeholders showed fairly strong support for this intervention, with 60% of respondents across all categories showing support at least to some extent.

176

B2 Introduce additional alert/information thresholds

Policy Measure Env. impact Soc. Impact Eco. Impact Cost Policy synergy Benefit to cost Included in policy options B2 Introduce additional alert/information thresholds
+ + (+) (+) + 0 0 0 0 0 0

(-) (+) Medium Policy Option II-3 Focus of measure: Define alert thresholds and information thresholds for all air pollutants as triggers for alerting the public and taking short-term action. Description of measure: This intervention would establish alert thresholds and information thresholds for some or all air pollutants that currently do not have alert thresholds or information thresholds, as triggers for alerting the public and taking short-term action. Stakeholder views: A targeted survey provided feedback on ‘to which extent would this specific intervention address the identified (related) shortcomings’.

Summary: Alert and information thresholds provide a trigger for alerting the public and developing short term action. Short-term action is expected to benefit air quality indirectly to a small extent. Better information (on all relevant air pollutants) for the public would enable citizens, in particular vulnerable groups to take more targeted and effective personal measures to reduce their exposure to harmful air pollution, thereby having a direct small positive impact on human health. This intervention is expected to have small direct administrative costs for the Commission and competent authorities.

B3 Revise definition of average exposure standards

Policy Measure Env. impact Soc. impact Eco. impact Cost Policy synergy Benefit to cost Included in policy options B3 Revise definition of average exposure standards (+) (+) (+) (+) (+) 0 0 (+) 0 0 0

(-) (+) High Sub-option I-5
Focus of measure: Expand the application of the exposure reduction targets (relative reduction in exposure).

177

Description of measure: The Ambient Air Quality Directives include average exposure obligations among the current provisions to regulate PM2.5 concentrations. These complement the emission limit value for PM2.5 by targeting average concentration values across larger areas. Accordingly, the Ambient Air Quality Directives set national PM2.5 exposure reduction targets to protect human health (Article 15). The reduction target is a percentage reduction based on the initial concentration. To determine the initial concentration, an average exposure indicator is used (an average level determined on the basis of measurements at urban background locations throughout the territory of a Member State and which reflects population exposure). This intervention explores whether the formulation of the average exposure reduction targets and obligations should be changed. According to Article 15 of Directive 2008/50/EC, the distribution and the number of sampling points on which the average exposure indicator for PM2.5 is based should reflect the general population exposure adequately. Annex XIV to Directive 2008/50/EC specifies Average Exposure Indicators (AEI) for PM2.5. The AEI is currently measured at urban background stations, which might not always be reflective of the general population exposure. The following variants are explored:
1. Introduce an exposure reduction target at regional or local level (rather than at national level only).
2. Broaden the “average exposure indicator” metric to include locations other than urban background (e.g. rural background locations).
3. Establish requirements for Member States to adopt air quality plans to meet exposure concentration obligations. Stakeholder views: A targeted survey provided feedback on ‘to which extent would this specific intervention address the identified (related) shortcomings’.

Summary: This intervention may improve the way that the average general population exposure reduction is monitored and addressed. The average exposure indicator is currently measured in urban background stations, which might not always be reflective of general population exposure. As a result, this measure is likely to provide better targeting of general air pollution exposure reduction measures, thereby contributing to further protection of public health from harmful air pollution and reducing the air quality cost to society. It could also improve the effectiveness of implementing mitigation measures. Direct costs estimated with this intervention are small administrative costs for the Commission and Member States.

178

B4 Introduce guidance on addressing exceedances

Policy Measure Env. impact Soc. impact Eco. impact Cost Policy synergy Benefit to cost Included in policy options B4 Introduce guidance on addressing exceedances (+) (+) (+) (+) 0 0 0 0 0 0 0

(-) (+) Medium Policy option II-1 Focus of measure: Provide guidance on the provisions concerning types of EU air quality standards and on the action to be taken in case of exceedance of different types of standards Description of measure: This intervention would include guidance on how to respond to exceedances in terms of suitable air pollution response measures in case of exceedances, and on types of plans to be used. It would aim to enable clearer coordination with the development and implementation of short-term action plans under Article 24 of Directive 2008/50/EC and air quality plans under Article 23 Directive 2008/50/EC by clarifying the information to be provided in short-term action plans and ensure the requirements under short-term air quality plans do not overlap with the requirements for air quality plans set in Annex XV to the Directive 2008/50/EC. Stakeholder views: A targeted survey provided feedback on ‘to which extent would this specific intervention address the identified (related) shortcomings’.

Summary: While varying circumstances across different EU member states are a challenge for developing effective guidelines, guidance could overall contribute towards better targeting of air pollution action, thereby contributing towards either more cost-effective response to exceedances or reducing the air quality cost on society by further protecting the general population from harmful air pollution. It is difficult to estimate indirect compliance and potential mitigation costs. Direct costs estimated with this intervention are small administrative costs for the Commission.

179

B5 Introduce limit values for additional air pollutants

Policy Measure Env. impact Soc. impact Eco. impact Cost Policy synergy Benefit to cost Synergies with other measures Included in policy options B5 Introduce limit values for additional air pollutants
++ + + + + 0 0 + - 0 -

(-) 0 Medium
Policy option II-2

Focus of measure: Establish limit values for additional air pollutants (i.e. for air pollutants currently subject to target values). Description of measure: The Fitness Check of the Ambient Air Quality Directives found that limit values have been more effective in facilitating downward trends than other types of air quality standards, such as target values. This intervention explores the establishment of limit values for additional air pollutants (i.e. for air pollutants currently subject to target values). Intervention options for additional limit values include: 1. Establish limit values also for air pollutants that tend to depend on transboundary precursors and /or annual variations in meteorology (e.g. as is the case for ozone); 2. Establish limit values also for air pollutants that tend to correspond to specific point source emissions (e.g. as is the case for most heavy metals); 3. Establish limit values also for air pollutants that tend to correspond to emissions from specific widespread practices (e.g. as is the case for most poly-aromatic hydrocarbons). Stakeholder views: A targeted survey provided feedback on ‘to which extent would this specific intervention address the identified (related) shortcomings’.

Summary: Limit values have proved most effective in reducing air pollutant concentrations. Introduction of limit values for all pollutants, where these would prove feasible, would strengthen the Ambient Air Quality Directive. Direct costs estimated with this intervention are medium administrative costs for the Commission, associated with the review of the Ambient Air Quality Directive as well as additional monitoring needs (which would depend on the selection of pollutants for which limit values would be defined). One reason for setting target values rather than limit values is to take account of the specific formation mechanisms, for example in the case of ozone (also due to a strong role of transboundary sources and annual variations in meteorology for this air pollutant).

180

2.3 Intervention area C: Actions when exceedances occur C1 Revise obligations for measures triggered by exceedances of air quality standards

Policy Measure Env. impact Soc. impact Eco. impact Cost Policy synergy Benefit to cost Included in policy options C1 Revise obligations triggered by exceedances (+) (+) (+) (+) 0 (+) 0 (+) 0 0 0

(+) Medium Policy option II-1 Policy option II-4 Focus of measure: Further specify the obligations to take measures to keep exceedance periods as short as possible. Description of measure: This intervention would maintain the obligation to set out “appropriate measures, so that the exceedance period can be kept as short as possible” while further specifying the ‘type of measures’ that competent authorities must take to ensure that exceedance periods can be kept as short as possible. The type of measures to consider will depend on the type of pollutant, the source of pollution, and other factors. To this purpose the revised Ambient Air Quality Directives would contain a checklist of relevant abatement measures that competent authorities can consider and select from. The measures set out currently in section B of Annex XV could be updated and applied to air quality plans. Competent authorities would have to demonstrate that they have considered all relevant measures in the checklist of measures and if they decided not to implement a relevant measure, this should be justified (unlike currently, where air quality plans are not required to include reasoning behind the measures adopted). This means that this intervention would provide for a systematic assessment of measures and strengthen the information requirements that competent authorities need to make available in air quality plans. This intervention would build on requirements of Article 23(1) of Directive 2008/50/EC linked to exceedances of limit or target values. It could be extended to the Exposure Concentration Obligation (ECO) and Average Exposure Indicator (AEI), should the revision lead to establishing requirements in case of not complying with those standards. The rationale behind this intervention is that air quality plans have often proven ineffective due to inadequate or not sufficiently ambitious measures to reduce air pollution to achieve compliance. Stakeholder views: A targeted survey provided feedback on ‘to which extent would this specific intervention address the identified (related) shortcomings’.

(+) Medium Policy option II-3 Focus of measure: Reformulate the term “as short as possible” including a defined time period. Description of measure: This measure would entail amending the text of Article 23 of Directive 2008/50/EC to define the specific time period within which competent authorities must bring emissions down below the exceedance threshold. This would replace the current wording “as short as possible”. This current provision is open to interpretation and therefore risks that exceedances remain systematic and persistent. In practice, since air quality plans must be prepared within two years from the exceedance at the latest, measures are often implemented only after three years at the earliest. Thus, the purpose of this intervention is to prompt competent authorities to take measures to reduce air pollution to a safe level in a timely manner. Where measures are implemented slowly, this intervention could contribute to ensuring that action is taken faster and that there is no room for different interpretations of what ‘as soon as possible’ means, as also voiced in the targeted stakeholder survey.

182

Stakeholder views: A targeted survey provided feedback on ‘to which extent would this specific intervention address the identified (related) shortcomings’.

Summary: Specifying a clear time period within which air quality standards have to be respected holds the potential to result in faster action which in turn can bring indirect positive benefits in terms of air quality and related impacts. Introducing a fixed timeframe will provide a maximum time span within which results have to be achieved, improving the speed of response rates in many cases. However, as there is no one-size-fits-all timeframe, there is a risk that a fixed timeframe will slow down action in some cases where compliance could be achieved before the end of the fixed term. There may also be effective long-term measures that cannot be fully implemented within the given timeframe. A fixed timeframe may also weaken previous interpretations of the term ‘as short as possible’ by the courts.

C3 Revise short-term action plans and air quality plans

Policy Measure Env. impact Soc. impact Eco. impact Cost Policy synergy Benefit to cost Included in policy options C3 Revise short-term action plans & air quality plans (+) (+) (+) 0 0 (+) 0 (+) 0 0 0

0 Medium Policy option II-1 Focus of measure: Require a clearer coordination between short-term action plans and air quality plans. Description of measure: This intervention consists of requiring clear coordination between the development and implementation of short-term action plans (under Article 24 of Directive 2008/50/EC) and air quality plans (under Article 23 and in Annex XV to Directive 2008/50/EC). It should be noted that this intervention is particularly relevant for Member States in which alert thresholds are exceeded but could apply to any Member State where there is a risk of exceeding limit or target values.
Coordination between short term action plans and air quality plans is not a requirement in the current Directive. As a result, not all Member States coordinate these. Since short term action plans and air quality plans may be under the responsibility of authorities at different levels

183

(for example, the former may be under the responsibility of local authorities, while the latter of regional authorities), coordination may require additional efforts.
Stakeholder views: A targeted survey provided feedback on ‘to which extent would this specific intervention address the identified (related) shortcomings’.

Summary: Coordination between short term action plans and air quality plans would lead to synergies among actions and avoid inefficiencies or inconsistencies. Small administrative costs may be incurred for Member State competent authorities related to coordination activities which are expected to be more than off-set by efficiency gains. According to several respondents to the targeted stakeholder survey, the revised Directive could require that short term action plans are included in air quality plans. Also, to facilitate this linkage between the two types of plans, the Ambient Air Quality Directives should include the minimum content that short-term action plans should contain.

C4 Introduce additional short-term action plans

Policy Measure Env. impact Soc. impact Eco. impact Cost Policy synergy Benefit to cost Included in policy options C4 Introduce additional short- term action plans + + + (+) + (+) (-) (+) 0/- (-) 0/-

- (+) Medium Policy option II-3 Focus of measure: Introduce an obligation for effective short-term action plans for each pollutant to prevent / tackle air pollution events. Description of measure: This intervention consists of introducing in the revised Ambient Air Quality Directives the obligation to adopt effective short-term action plans for all pollutants to prevent and tackle pollution events.
Directive 2008/50/EC requires that action plans are drawn up indicating the measures to be taken in the short term “where there is a risk of an exceedance of one or more alert thresholds” (in order to reduce the risk of the duration of such an exceedance). However, alert

184

thresholds as defined in Annex XII of Directive 2008/50/EC only exist for NO2, SO2 and O3, and therefore short-term action plans are not required for other pollutants such as PM10.59
Stakeholder views: A targeted survey provided feedback on ‘to which extent would this specific intervention address the identified (related) shortcomings’.

Summary: An obligation for effective short-term action plans for each pollutant would prompt further action to bring emissions and concentrations down compared to the current situation, thus expected to benefit air quality and protect in particular sensitive groups from immediate health risks. Additional administrative burden is expected from this intervention as it imposes new requirements to Member State competent authorities. Risks linked to this intervention have to do with time-lag and separation of source from pollution. Short-term action plans may be effective only to a limited extent where pollution episodes cannot be influenced by local measures or in case of secondary pollutants for which it is not straight forward to identify immediate measures.

C5 Introduce a requirement to update air quality plans

Policy Measure Env. impact Soc. impact Eco. impact Cost Policy synergy Benefit to cost Included in policy options C5 Introduce requirement to update air quality plans (+) (+) (+) (+) 0 (+) 0

## (+)

## 0

(+) Medium Policy option II-3 Focus of measure: Mandate regular updates of air quality plans. Description of measure: This measure would introduce an obligation for competent authorities to update air quality plans at regular intervals to keep exceedance periods as short as possible. Specific frequency of the update would take into account the administrative burden such updates entail. Based on feedback received from the Targeted Stakeholder

0 High Policy option II-1 Focus of measure: Establish a requirement for Member States to involve specific actors in air quality plan development and to specify coordination arrangements for the development and implementation of air quality plans. Description of measure: This measure would require Member States to involve all relevant actors in the drafting of air quality plans and coordinate better with these. Actors may include national/regional/local competent authorities, sectoral representatives from polluting industries, research institutes, civil society and local citizens. To this purpose, the revised Ambient Air Quality Directives should include the following concerning the preparation of air quality plans (1) a requirement for consulting and involving government authorities at various levels, and (2) a new ‘public participation’ clause for the development of air quality plans. The revised Directive should specify which aspects of the planning process should be open to public consultation and what this should involve. The problem that this measure is trying to address is that since there are no requirements on how to allocate roles and responsibilities in air quality plans, cooperation between government authorities at various levels is not a given. This can lead to insufficient action being taken by public authorities or to a mismatch of action, and therefore to air quality plans and measures being insufficient, inefficient and/or ineffective. In addition, while air quality remains a top environmental concern for EU citizens, citizens are not systematically consulted in the development of air quality plans. Stakeholder views: A targeted survey provided feedback on ‘to which extent would this specific intervention address the identified (related) shortcomings’.

Summary: This measure seeks to improve the involvement of all relevant actors in the design and implementation of air quality plans. This may be done by possibly adding a

187

requirement for consulting and involving government authorities at various levels, and by introducing a new ’public participation’ clause for the development of air quality plans.

D2 Introduce a ‘one zone, one plan’ requirement

Policy Measure Env. impact Soc. impact Eco. impact Cost Policy synergy Benefit to cost Included in policy options D2 Introduce a ‘one zone, one plan’ requirement 0 0 0 0 0 0 0 0 0 0 0

0 Low Sub-option II-1a Focus of measure: Introduce a requirement for Member States to harmonise air quality plans and air quality zones (and require a ‘one zone, one plan’ approach). Description of measure: This measure would further define the requirements for drawing air quality plans in Article 23 of the Directive 2008/50/EC to require that one zone has to fully overlap with one plan (and hence avoiding zones with multiple plans and plans for multiple zones). This measure aims to increase the effectiveness of the Ambient Air Quality Directives by tackling the current mismatch between the zones of air quality monitoring and air quality plans. Stakeholder views: A targeted survey provided feedback on ‘to which extent would this specific intervention address the identified (related) shortcomings’.

Summary: This measure aims to increase the effectiveness of the Ambient Air Quality Directives by tackling the current mismatch between the zones of air quality monitoring and air quality plans. However, the benefits and added value of this intervention are unclear while it would generate some costs (and considerable administrative burden). Overall it is unclear what the added value of this intervention would be and a global approach does not seem helpful as air quality plans and air quality zones are very specific to local conditions. Arguments against this intervention in the Targeted Stakeholder Survey revolve around changes that would be needed in terms of governance / responsibilities as well as around additional administrative burden that the intervention would lead to.

188

2.6 Intervention area E: Enforcement tools
E1 Introduce minimum levels for financial penalties

Policy Measure Env. impact Soc. impact Eco. impact Cost Policy synergy Benefit to cost Included in policy options E1 Introduce minimum levels for financial penalties
(+)
(+)
(+)
(+)
0
(+)
0 (+) (-)
0

(-)

(-)

(+) Medium / High Policy option II-4 Focus of measure: Introduce minimum levels for financial penalties decided at national level. Description of measure: This measure aims to expand the current provisions on penalties in the Ambient Air Quality Directives (Article 30 of Directive 2008/50/EC and Article 9 of Directive 2004/107/EC) to specify the magnitude of the financial penalties to be paid. In cases of failure to comply with air quality standards by establishing a minimum level for such. These penalties would be directed to competent authorities as well as industry or other private entities and should lead to penalties or sanctions that are high enough to be effective and dissuasive. The number of continued exceedance situations can be seen as an indication that Member State penalties are not sufficiently effective, proportionate nor dissuasive, with the effect that the legislation has not been adequately implemented. Further, currently financial sanctions differ from Member State to Member State leading to leading to discrepancies in terms of level of penalties and their application across the EU. While penalties are to be laid down by Member States, there is potential for more clearly framing the use and scope of penalties in the Ambient Air Quality Directives following the examples of other EU legislation. Stakeholder views: A targeted survey provided feedback on ‘to which extent would this specific intervention address the identified (related) shortcomings’.

Summary: Effective minimum penalty levels should discourage competent authorities and industry or other private entities from breaching provisions of the Directives or measures adopted pursuant to the Directives, thus indirectly benefiting air quality, ecosystems and health. If effective, it would lead to competent authorities and industry implementing more measures to avoid breaches (and therefore avoid the high fines). This would indirectly generate additional costs for these actors, though related to achieving compliance. The

189

additional administrative burden of clarifying levels of financial penalties is low and would facilitate their implementation. The risks for implementation have to do with determining penalty levels applicable across the EU and, more indirectly, with difficulties with enforcement of breaches.

E2 Introduce right to health damage compensation

Policy Measure Env. impact Soc. impact Eco. impact Cost Policy synergy Benefit to cost Included in policy options E2 Introduce right to health damage compensation (+)
(+) (+)
(+) 0 (+) 0 (+) - 0 -

0

(+) Medium Policy option II-4 Focus of measure: Introduce an explicit provision that provides a right to compensation for damage to health caused by air pollution.
Description of measure: This measure would introduce an explicit provision within the Ambient Air Quality Directives on the right to compensation for damage to health caused by breaches of the Ambient Air Quality Directives. The principle of state liability allows for individuals to seek compensation under certain conditions for harm suffered as a result of Member State non-compliance with EU law.61 Such a provision would clarify and facilitate compensation for harm suffer to health from air pollution. The reason for this measure is that while there is overwhelming epidemiologic evidence on the negative health impacts of air pollution on the population, exceedances still take place (albeit the frequency, extent and magnitude of these have generaly improved since 2008) and damages linked to these are not always addressed sufficiently. Stakeholder views: A targeted survey provided feedback on ‘to which extent would this specific intervention address the identified (related) shortcomings’.

61 The application of this principle of state liability to breaches of Member States’ obligations pursuant to Articles 13 and 23 under the Ambient Air Quality Directives is the subject of a preliminary reference currently before the Court of Justice of the EU in Case C-61/21, Ministre de la Transition écologique and Premier ministre. See Annex 12 for more detail on the case.

190

Summary: This measure would work as an incentive for competent authorities and industry/business to implement more effective measures, which in turn would benefit air quality, health and ecosystems. This measure, if implemented, would require competent authorities and/or industry (polluters) to pay compensation to those who have suffered damage to health from air pollution and would therefore carry mitigation costs for those who are held accountable for breaches of air quality standards. It would also carry administrative burden for competent authorities and/or industry (polluters) as they would need to put in place and manage the compensation scheme and deal with a potentially increasing number in lawsuits by citizens / civil society, though only in case of continued non-compliance. Implementation challenges include the difficulty to prove the causal link between pollution and long-term health effects and the question of accountability (who is held responsible).

E3 Introduce a fund to be fed by penalties paid

Policy Measure Env. impact Soc. impact Eco. impact Cost Policy synergy Benefit to cost Included in policy options E3 Introduce a fund to be fed by penalties paid 0/(+) 0/(+) 0/(+) 0/(+) 0 (+) 0 0 0/(+) 0 0/+

(-) 0/(+) Low Sub-option II-4a

Focus of measure: Set up a fund to be fed by the payment of penalties which can be used to compensate material damage or finance air quality measures. Description of measure: This measure consists of setting up a “clean air fund” to be fed by the payment of penalties when the rules established by the Ambient Air Quality Directives, or possibly other rules addressing air pollution, are infringed. It would be used to compensate victims of air pollution as well as to finance air quality measures. The fund could be established either at EU-level (an EU-wide fund) or at national level (with each Member State having their own fund). Stakeholder views: A targeted survey provided feedback on ‘to which extent would this specific intervention address the identified (related) shortcomings’.

191

Summary: A dedicated fund would make available funding for compensation for health damage suffered and facilitate access to funding of the implementation of mitigation measures (leading to measures being more readily implemented). However, it could also lead to competent authorities using the fund to finance measures that they would implement in any case, without leading to ‘more’ (i.e. additional) measures being implemented, which is a risk the governance of the fund would have to address. Setting up and administering such a fund will generate additional burden. Risks for implementation include a potential conflict of interest in the case the authority that has to pay also administers the fund and alignment with national budgetary rules. The organisation of the fund could provide safeguards to avoid that the budget from which the penalty is paid into the fund is not the one benefiting from it.

E4 Introduce an explicit ‘access to justice’ provision

Policy Measure Env. impact Soc. impact Eco. impact Cost Policy synergy Benefit to cost Included in policy options E4 Introduce access to justice provision (+)
(+)
(+)
(+)
0 (+) 0

(+) (-)
0 (-)

0 0 High Policy option II-4

Focus of measure: Introduce an explicit provision in the Ambient Air Quality Directives that grants the public concerned ‘access to justice’. Description of measure: This measure introduces a new explicit provision on ‘access to justice’ in the Ambient Air Quality Directives. The Fitness Check of the Ambient Air Quality Directives found that enforcement action by civil society actors in front of national courts has proven to be important to accelerate downward trends for air pollution. This has been confirmed by the Inception Impact Assessment, which notes that “the effectiveness of legal enforcement action by civil society is linked to the functioning of access to justice at national level”.62 However, studies have shown that rules on access to justice rules vary widely between Member States and that there exist still significant hurdles to effective access to justice at national level.63

62 COM (2020), Inception impact assessment - Ares(2020)7689281 (accessed: 04.08.2022) 63 For example: 2013 access to justice report on the Implementation of Articles 9.3 and 9.4 of the Aarhus Convention in the Member States of the European Union and 2019 Milieu Study on EU implementation of the Aarhus Convention in the area of access to justice in environmental matters. (accessed: 10.06.2022)

192

Stakeholder views: A targeted survey provided feedback on ‘to which extent would this specific intervention address the identified (related) shortcomings’.

Summary: There is a gap in the Ambient Air Quality Directives with regard to ‘access to justice’ and including such a provision in the legislation would be a coherent step, in line with other environmental Directives,64 Article 47 of the EU Charter of Fundamental Rights, the Aarhus Convention65 and the case law of the Court of Justice of the EU.66 Public judicial enforcement of the obligations under the Ambient Air Quality Directives has so far already lead to multiple national rulings (in several Member States) mandating national authorities to take action to improve air quality. Introducing an explicit provision would enable such action by citizens that are currently unable to do so because of strict national procedural requirements.67 In turn, this would indirectly benefit air quality and human health as a whole. Additional administrative costs for Member States (probably central / national government) and industry may occur as an increase in lawsuits may be expected; this would largely depend on whether national authorities have already taken the necessary measures to comply with the Aarhus Convention and the relevant case law of the Court of Justice of the EU. The implementation of the intervention carries risks in terms of capacity for Member States to deal with additional legal claims.

64 For example: Article 6(2) of Directive 2003/4/EC, Article 13 of Directive 2004/35/EC, Article 11 of Directive 2011/92/EU, Article 25 of Directive 2010/75/EU and Article 23 of Directive 2012/18/EU.
65 UNECE (1998), Convention on access to information, public participation in decision-making and access to justice in environmental matters (accessed: 10.06.2022) 66 For an overview see: Commission Notice on Access to Justice in Environmental Matters (accessed: 10.06.2022) 67 See Annex 12 for an illustrative overview of clean air cases before national courts.

193

2.7 Intervention area F: Information to the public
F1 Revise provisions related to up-to-date data

Policy Measure Env. impact Soc. impact Eco. impact Cost Policy synergy Benefit to cost Included in policy options F1 Revise provisions related to up-to-date data 0 (+) 0 0 (+) 0 0 0 0 0 0

0 Medium / High Policy option IV-1

0 Medium Policy option IV-2

Focus of measure: Require Member States to provide specific health / and health protection information to public as soon as exceedances occur. Description of measure: This intervention would require Member States to provide information to the public as soon as exceedances of alert thresholds occur. The issue that this intervention is trying to solve is that currently when alerts are made public, it is often too late to protect the health of the population because pollution peaks often do not last long. A standardised approach to providing information about the negative health effects in a simple, understandable form may prove useful for considering under this intervention. Stakeholder views: A targeted survey provided feedback on ‘to which extent would this specific intervention address the identified (related) shortcomings’.

Summary: Information on health (protection) would allow citizens to make decisions that may impact on their health such as deciding not to exercise outdoors or opting for a cleaner transport route. Ensuring that information is provided to allow citizens to take timely action would increase the effectiveness of information provided, whilst the costs are considered negligible since relevant information and the systems to provide it are already in place.

195

F3 Introduce use of specific communication channels

Policy Measure Env. impact Soc. impact Eco. impact Cost Policy synergy Benefit to cost Included in policy options F3 Introduce use of specific communication channels 0 0 0 0 (+/-) 0 0 0 0 0 0

0 Low Sub-option IV-2a

Focus of measure: Mandate specific communication channels with citizens, including user- friendly tools for public access to air quality and health risks information.
Stakeholder views: A targeted survey provided feedback on ‘to which extent would this specific intervention address the identified (related) shortcomings’.

Summary: Obliging competent authorities to use a set of information channels would lead to a better, and consistently informed public with indirect benefits on health, however given the case care has to be taken on which channels to define for use. The cost of developing (in particular where these are not currently in place) specific, high-tech channels may be more costly, which may divert resources from other, more productive, means.

Description of measure: This intervention would mandate the use of specific user-friendly communication channels to reach out to citizens (for example, smartphone apps, social media, text messages, forecasts on television (similar to weather forecasts)) so that citizens have access to air quality data and information related to health risks. The issue this intervention is trying to solve is that citizens do not always know where to access (reliable) air quality information and that governments do not know how to best provide information. Tools and the quantity and quality of information provided to citizens varies between Member States.

196

F4 Introduce requirements for harmonised air quality index

Policy Measure Env. impact Soc. impact Eco. impact Cost Policy synergy Benefit to cost Included in policy options F4 Introduce requirements for harmonised air quality index 0 (+) 0 0 (+) 0 0 0 0 0 0

- 0 Medium Policy option IV-3 Focus of measure: Require Member States to use harmonised air quality index bands. Description of measure: This intervention consists of including a provision in the Directive 2008/50 to require Member States to use harmonised air quality index bands, namely those used in the European Air Quality Index. This way a one-size-fits-all where everyone adopts the same index is avoided, acknowledging that different countries and regions have their own characteristics which make different pollutants relevant. The problem that this intervention is aiming to solve is the current absence of a common metric used for publicised air quality indices. At the moment Member States (and even regions within in some cases) have different air quality indices whose bands and thresholds differ from the European Air Quality Index provided by the European Environmental Agency. This often means that the same data is presented in different ways in different locations. Although there is no consensus on whether and how air quality indices can be harmonised, what is known (from the study “Strengthening of air quality monitoring, modelling and plans under the Ambient Air Quality Directives”) is that there is not much support for all Member States adopting the European Air Quality Index. As such adopting the bands alone seems the most feasible compromise which has obtained wide support in the stakeholder consultation activities. Stakeholder views: A targeted survey provided feedback on ‘to which extent would this specific intervention address the identified (related) shortcomings’.

Summary: Consistency in the information provided to citizens will aid clarity and uniformity in the opportunity provided to all EU citizens to take action to reduce their exposure with indirect benefits for health. However, there are concerns that the European Air Quality Index is not effective (e.g. around its ability to represent multi-pollutant effects), and that complete harmonisation may restrict the ability of Member States to tailor advice and information to the specific situation in each Member State. The intervention will increase administrative burden for competent authorities (regional or national) as it will require these to adapt their index bands.

197

2.8 Intervention area G: Assessment regimes
G1
Revise rules related to indicative sampling points

Policy Measure Env. impact Soc. impact Eco. impact Cost Policy synergy Benefit to cost Included in policy options G1 Revise rules related to indicative sampling points (+) (+) (+) 0 0 (+) 0 (+) 0 0 0

0 High Policy option III-1

0

Medium / High Policy option III-1 and Policy option III-5

Focus of measure: Make the use of air quality modelling mandatory as part of air quality assessment (in some circumstances). Description of measure: Modelling techniques can provide valuable information to supplement fixed measurements. Observations from fixed stations are limited to the sampling locations itself whereas modelling systems most often provide air quality maps with a full spatial coverage that can be used to derive specific indicators. Modelling can also help to disentangle the origin of the observed concentrations (source apportionment, long range transport) or extrapolate into the future (short term forecasts, future projections). Several variants exist for this intervention – related to the possible use of air quality modelling: (1) For short term air quality forecasting (up to a few days ahead); (2) For assessment of air quality for compliance checking purposes; (3) For air quality near real time mapping and informing the public; (4) For evaluation of monitoring network design; (5) For estimation of population exposure and exceedance situation indicators; (6) For source apportionment estimations; (7) For assessment of long-range air pollutant transport; (8) For future projections in support of air quality management and planning; (9) As alternative to fixed monitoring (when placing such monitoring in line with the Directive is not possible).

68 As expressed in their internal Working Group document on suggestions for the Revision of the Ambient Air Quality Directives of December 2021.

199

Stakeholder views: A targeted survey provided feedback on ‘to which extent would this specific intervention address the identified (related) shortcomings’.

Summary: Air quality modelling improves air quality monitoring and assessment, thus allowing for a better understanding of air quality concentrations, supporting a more effective and more targeted air quality management. The administrative burden may increase for competent authorities (to meet the reporting requirement). The increase is likely to depend on the current modelling capability and practices within each Member State. There are risks for implementation linked to technical capacity and potential lack of modelling guidance and/ or lack of resources for training and capacity building. There is also a risk that Member States may view the introduction of a mandatory requirement of modelling as a reason to reduce their monitoring network. There is strong support across all stakeholder types for the mandatory use of modelling for most of the nine use case variants in at least some instances. Some respondents, however, explained further that modelling should be (strongly) recommended in most of these use cases but only made mandatory for all Member States in one case, i.e. for future projections in support of air quality management and planning. The option of introducing requirements for the use of modelling for compliance checking purposes was the least favoured option among public authorities. The Forum for Air Quality Modelling (FAIRMODE) recommends the use of modelling for assessment purposes, forecasting and public information purposes, source apportionment and planning purposes, making it mandatory for air quality planning, exposure calculations, and short-term forecast.69

69 Thunis P., Janssen S., Wesseling J.,Piersanti A., Pirovano G., Tarrason L., Martin F., S. Lopez-Aparicio, Bessagnet B., Guevara M., Monteiro A., Clappier A., Pisoni E., Guerreiro C., González Ortiz A., Recommendations for the revision of the ambient air quality directives (AAQDs) regarding modelling applications.

200

G3 Revise rules for regular review of AQ assessment

Policy Measure Env. impact Soc. impact Eco. impact Cost Policy synergy Benefit to cost Included in policy options G3 Revise rules for regular review of AQ assessment 0 0 0 0 0 0 0 0 0 0 0

(-) 0 Low Sub option III-1a

Focus of measure: Require a regular review of the assessment regime following clear criteria defined in the Directive. Description of measure: Regular review of the assessment regime is expected to ensure that the assessment techniques for air quality evolve with scientific advancements and knowledge. It also allows for improved and increased evidence on air quality including the use of models and more efficient monitoring networks. This would require the amendment of existing articles and Annex II point B to include set criteria. In addition, the interval at which a review should be done was queried with the options of every ten, five, three or one year(s). Stakeholder views: A targeted survey provided feedback on ‘to which extent would this specific intervention address the identified (related) shortcomings’.

Summary: This intervention would require Member States to follow set criteria in their reviews of their assessment regime which rely on monitoring and/or modelling data. This would provide a more harmonised review of air quality assessment across Europe leading to a more transparent and coherent view of air quality status for wider public access. All Member States have ready access to fixed term monitoring, and most have modelling capability, so our expert view is that the costs for this intervention are insignificant. Administrative burden may be significant if the period for review is annual (stakeholder respondents favored the retention of five year reviews).

201

2.9 Intervention area H: Number and typology of sampling points H1 Revise minimum number of sampling points

Policy Measure Env. impact Soc. impact Eco. impact Cost Policy synergy Benefit to cost Included in policy options H1 Revise minimum number of sampling points + + + 0 0 0 0 +/- 0 0 0

0

Medium Policy option III-1

Focus of measure: Change the minimum number of sampling points that are required per air quality zone. Description of measure: The minimum number of sampling points per air quality zone for each pollutant should be revised with latest scientific knowledge. Possibilities to which extent would the below specific interventions address the above identified shortcomings include: (1) Increase the minimum number of sampling points for all pollutants and all zones; (2) Increase the minimum number of sampling points for some pollutants; (3) Increase the minimum number of sampling points for some zones; (4) Decrease the minimum number of sampling points for all pollutants and all zones; (5) Decrease the minimum number of sampling points for some pollutants; (6) Decrease the minimum number of sampling points for some zones; (7) Require a minimum of 2 sampling points per zone per pollutant (i.e. to monitor both hotspots and background concentration levels); (8) Establish a minimum number in the vicinity of point sources in view of emission densities; (9) Establish a minimum number of sampling points for measuring pollution hotspots specifically; (10) Establish a minimum number of sampling points for measuring population exposure. Stakeholder views: A targeted survey provided feedback on ‘to which extent would this specific intervention address the identified (related) shortcomings’.

Summary: Changing the minimum number of sampling points required has potential for an increase or decrease in monitoring for the assessment of air quality (a reduction in the

202

minimum number of monitoring stations would be detrimental to air quality, public health, ecosystems and costs to society but lessen administrative burden, while an increase in the minimum number of stations would have a positive impact on air quality, health and ecosystem but increase administrative burden). Costs arising from an increased number of stations would be incurred from greater laboratory analysis, and the additional staff needed for servicing and maintenance and data management. There is little/no support from stakeholders for any decrease in the minimum number of sampling points, while an increase of monitoring stations was favored for at least some pollutants and with a minimum to measure population exposure. The network of National Air Quality Reference Laboratories also favors an increase of sampling points and suggests as well the removal of the possibility to reduce the number of sampling points if fixed measurements are supplemented with indicative measurements.70 While additional monitoring is associated with high costs, many Member States report monitoring above the current required number of sampling locations, and therefore in practice, an increase in monitoring required is overall beneficial.

H2 Simplify combined PM10/PM2.5 monitoring

Policy Measure Env. impact Soc. impact Eco. impact Cost Policy synergy Benefit to cost Included in policy options H2 Simplify combined PM10/PM2.5 monitoring (+) (+) 0 0 0 0 0 (+) 0 0 0

0 High Policy option III-1

Focus of measure: The minimum number of sampling points for measuring PM10 and PM2.5 will be considered independently from each other. Description of measure: This intervention de-couples of the current minimum number of sampling points for PM10 and PM2.5, which should be set independently and cannot substitute one another. Stakeholder views: A targeted survey provided feedback on ‘to which extent would this specific intervention address the identified (related) shortcomings’.

70 As expressed in their internal Working Group document on suggestions for the Revision of the Ambient Air Quality Directives of December 2021.

203

Summary: This intervention de-couples the current minimum number of sampling points for PM10 and PM2.5, which should be set independently and cannot substitute one another. PM2.5 is a key pollutant for public health risk assessment. Given its important impacts on mortality and morbidity, it is essential for it to be considered and assessed independently from PM10. Clarifying and giving more focus on the assessment of this pollutant in the revised Ambient Air Quality Directive would bring benefit to driving action in areas of exceedance to improve public health protection. Many Member States have already increased their sampling of PM2.5 so in practice this intervention is unlikely to involve large costs, though for those Member States who monitor at minimum levels only, costs may be significant as those for new monitoring samplers are often high and comes with on-going maintenance costs. Public Authorities report no real increase in administrative burden to monitor PM10 and PM2.5 separately. There is a time lag associated with this intervention (to establish the new sites) and this may risk air quality in the short term. Additional staff is needed to support sampler operation and data management.

H3 Simplify the definitions of sampling points types

Policy Measure Env. impact Soc. impact Eco. impact Cost Policy synergy Benefit to cost Included in policy options H3 Simplify the definitions of sampling points types 0 0 0 0 0 0 0 0 0 0 0

0

Low Sub option III-1b

Focus of measure: Simplify the definitions of types of monitoring station and/or sampling point locations - and only differentiate for them to distinguish between hotspots or background concentrations. Description of measure: Currently station classification includes a number of categories such as urban, suburban, rural, industrial, roadside etc. station classification could be simplified to identify sites as hotspots or background locations. Stakeholder views: A targeted survey provided feedback on ‘to which extent would this specific intervention address the identified (related) shortcomings’.

204

Summary: Currently station classification includes a number of categories such as urban, suburban, rural, industrial, traffic, background. Station classification could be simplified to identify sites only as hotspots or background locations. This may enable identifying a key source of pollution currently missing in the classification which is that due to residential combustion. As this intervention is a desk task to reclassify the current sites it is unlikely to have any real impact on administrative burden. To be effectively implemented, this intervention would require guidance. However, a more simplified classification risks loss of clarity and misunderstanding on the site differences and the main sources of pollution. The full reporting of site meta data under the IPR71 and e-reporting by all Member States and clarification of terms further in the Ambient Air Quality Directive could greatly help to address this shortcoming.

71 COM (2013), IPR Guidance part I and Part II (accessed 10.06.2022)

205

2.10 Intervention area I: Continuity of sampling points
I1 Introduce obligations to maintain sampling points

Policy Measure Env. impact Soc. impact Eco. impact Cost Policy synergy Benefit to cost Included in policy options I1 Introduce obligations to maintain sampling points (+) (+) (+) 0 (+) 0 0 0 0 0 0

0 0 High Policy option III-2

Focus: Specify that sampling points with exceedances of limit values for any of the pollutants measured under the Ambient Air Quality Directives should be maintained for a defined number of years. Description of measure: Flexibilities in the Ambient Air Quality Directives enable monitoring sites to close or be relocated (except for PM10 if exceeding limit values), but this disrupts trend analysis and causes uncertainty in areas of exceedance. This intervention would prevent sampling point closure within a defined number of years following site establishment. Possibilities under which circumstances can relocations of sampling points take place include: (1) Due to requirements of local spatial development; (2) If and when siting criteria are no longer met (macro-scale siting or micro-scale siting); (3) If overlap between monitoring at ‘old’ and ‘new’ sampling point is guaranteed and reported (for a defined time period ensure monitoring at both locations to assure calibration) Stakeholder views: A targeted survey provided feedback on ‘to which extent would this specific intervention address the identified (related) shortcomings’.

Summary: Providing clarity on the circumstances when sampling points may be relocated would reduce flexibility to close stations but allow for increased datasets for pollutant trend analysis. Requiring a set timeframe for the operation and maintenance of sampling points with exceedances of limit values for any of the pollutants under the Ambient Air Quality Directives would result in better datasets for assessment and trend analysis. This would in most cases be a prerequisite for more effective and more targeted air quality management.

206

I2 Introduce obligations to monitor long-term trends

Policy Measure Env. impact Soc. impact Eco. impact Cost Policy synergy Benefit to cost Included in policy options I2 Introduce obligations to monitor long-term trends (+) (+) (+) 0 0 0 0 0 0 0 0

(-)

0 Low Sub-option III-2a

Focus of measure: Include the requirement to monitor long-term trends if fixed monitoring stations are discontinued (by assessing air quality via indicative measurements or air quality modelling), to not disrupt trend analysis. Description of measure: Currently, flexibilities in the Ambient Air Quality Directives enable monitoring sites to close or be relocated, but this disrupts trend analysis. Under the circumstances where stations are discontinued a requirement could be introduced to continue to monitor for long-term trends using indicative measurements or modelling. Stakeholder views: A targeted survey provided feedback on ‘to which extent would this specific intervention address the identified (related) shortcomings’.

Summary: Including a requirement to monitor long-term trend in the cases of relocation of fixed monitoring stations would allow for increased datasets for pollutant trend analysis. Assessing long-term trends in pollution data is important for the assessment and management of air quality. There is significant benefit to scientific understanding and policy development to protect health and the environment to have access to a long-established network of monitors. Costs for this intervention depend on the variant. Administrative burden and costs of monitoring could increase as the amount of fixed monitoring stations would remain the same, but it may be required to increase indicative measurements at all previous fixed measurement locations for long-term trend monitoring and analysis. However, where fixed monitoring stations could be replaced by indicative monitoring or modelling a cost saving is likely. For this intervention to be successful, it is important to align with those proposed interventions with the objective of improving quality of indicative monitoring and modelling.

207

I3 Introduce a protocol for relocated sampling points

Policy Measure Env. impact Soc. impact Eco. impact Cost Policy synergy Benefit to cost Included in policy options I3 Introduce a protocol for relocated sampling points (+) 0 0 0 (+) 0 0 0 0 0 0

- 0

Medium Policy option III-2

Focus of measure: Establish a protocol to follow should a sampling point have to be re- located due to, for example, infrastructure development or changes in the assessment regimes. Description of measure: Currently, flexibilities in the Ambient Air Quality Directives enable monitoring sites to close or be relocated, but this disrupts trend analysis. Whenever the circumstances of station discontinuation or sampling point relocation due to infrastructure development or changes in the assessment regime arise, a protocol establishing the requirements for such change should serve as guidance. Stakeholder views: A targeted survey provided feedback on ‘to which extent would this specific intervention address the identified (related) shortcomings’.

Summary: A protocol could include an assessment of site representativeness, co-location of monitoring for a minimum time period, to assist in the assessment of data quality for trend analysis from the old and new sampling points and hence increase robustness and transparency especially when areas are in exceedance. This intervention, while helpful for greater assessment harmonisation is likely to have little impact on air quality and other indicators. The costs for this intervention are low. Although, reduced flexibility to relocate samplers when necessary, it may risk increased administration burden on Member States to find an alternative monitoring location.

208

2.11 Intervention area J: Siting of sampling points
J1 Revise macro-scale siting of sampling points

Policy Measure Env. impact Soc. impact Eco. impact Cost Policy synergy Benefit to cost Included in policy options J1 Revise macro-scale siting of sampling points (+) (+) (+) 0 (+) 0 0 0 0 0 0

0

0

Medium Policy option III-4

Focus of measure: Further clarify (and reduce flexibilities related to) the micro-siting criteria for sampling points. Description of measure: This intervention has the following variants: (1) Harmonise the micro-scale siting criteria laid down in Annex III and Annex VIII of Directive 2008/50/EC and Annex III of Directive 2004/107/EC – aligning with 2008/50/EC provisions; (2) Clarify whether micro-scale siting criteria are applicable to sampling points for indicative measurements in addition to sampling points for fixed measurements; (3) Clarify the flexibility related the unrestricted flow around the inlet of sampling points. (4) Clarify the flexibility related to the height of the inlet of sampling points; (5) Clarify the flexibility related to the distance to the kerbside (or other metrics) of traffic- oriented sampling points. Stakeholder views: A targeted survey provided feedback on ‘to which extent would this specific intervention address the identified (related) shortcomings’.

Summary: Revisions to micro-siting criteria which also apply to indicative monitoring may have an indirect benefit to society costs due to an indirect improvement on public health.

210

Where new indicative monitoring is being planned this intervention may give access to a higher quality monitoring dataset to assist air quality assessment, underpinning air quality action. There is a low administrative burden, unless the intervention leads to the disqualification of existing sites (in which case the administrative burden would be high). Costs are relatively low, particularly if this intervention does not result in the disqualification of established long-term sampling locations. The mostly favoured sub variant refers to the clarification regarding the flexibility related to the distance to the kerbside (or other metrics) of traffic-oriented sampling points. In this sense, the most concern raised by stakeholders was on the micro-siting criteria for sampling points is related to traffic sites, particularly in urban areas. However, these are complex environments with pollution concentrations varying in small micro-environments. Some level of flexibility is needed to local monitoring network managers to ensure monitoring effectiveness and efficiency.

J3 Introduce obligation for spatial representativeness

Policy Measure Env. impact Soc. impact Eco. impact Cost Policy synergy Benefit to cost Included in policy options J3 Introduce obligation for spatial representativeness (+) (+) (+) 0 (+) 0 0 (+) 0 0 0

0

Medium Sub option III-3d

Focus of measure: Introduce the concept of a spatial representative area which should be estimated (and reported) for each sampling point (irrespective of exceedances being measured or not). Description of measure: For every sampling location, a spatial representativeness (SR) area should be estimated and reported. This area of representativeness is an essential indicator of the sampling location. A Tiered approach is available to assess SR of monitoring sites:  Tier 1: assessment based on expert judgement;  Tier 2: assessment based on proxy data or indicative measurement campaigns;  Tier 3: assessment based on fit-for-purpose modelling;  Tier 4: assessment based on combination of modelling and indicative monitoring.

211

Stakeholder views: A targeted survey provided feedback on ‘to which extent would this specific intervention address the identified (related) shortcomings’.

Summary: The concept of an SR area helps to clarify and harmonize air quality assessment based on monitoring data. It serves multiple purposes in this process: assessment of population exposure and exceedance situation indicators based on the monitoring data, monitoring network design and selection of stations for model validation and data assimilation. When modelling capacity is available higher Tier methods are rather straightforward to apply. Stakeholders indicate that there is a clear need for better definition for spatial representativeness and it would be useful to introduce this concept to the Ambient Air Quality Directives in order to ensure comparability between Member States.

212

2.12 Intervention area K: Data quality
K1 Revise AQ monitoring data quality objectives

Policy Measure Env. impact Soc. impact Eco. impact Cost Policy synergy Benefit to cost Included in policy options K1 Revise AQ monitoring data quality objectives
(+) (+) (+) 0 0 0 0 0 0 0 0

- 0

Medium Policy option III-4

Focus of measure: Further define the data quality requirements for sampling points / measurements used for air quality assessments. Description of measure: To further define data quality including measurement uncertainty and data capture. Variants for this intervention include:
(1) Further align data aggregation requirements to be met for specific periods (e.g. hourly, daily, 8-hour or annual) or the whole year; (2) Further align the data coverage (time coverage and data capture) requirements for all air pollutants; (3) For ozone, align data coverage requirements for both for the full calendar year and for the period of April to September, as well as for the AOT40 indicator; (4) For indicative measurements, set separate data coverage requirements for annual mean values and for short-term mean values; (5) For calibration and validation of air quality modelling, introduce specific data quality requirements for sampling points / measurements (that are less strict than those used for air quality assessments). Stakeholder views: A targeted survey provided feedback on ‘to which extent would this specific intervention address the identified (related) shortcomings’.

Summary: To make the full use of available data a protocol/guidance specifying appropriate methods for assessing compliance and estimating statistical parameters to account for low data coverage or significant data losses should be published. This intervention would improve data quality requirements for sampling points which is likely to increase robustness of data and may supplement evidence for trend analysis and modelling. This may lead to indirect improvements in air quality, health and ecosystems which may indirectly reduce costs to society as clarity is provided over the use of data. The costs for this are low or may even be a

213

cost saving as administrative burden may reduce as modelling is likely to cost less than additional fixed or indicative measurements. The most favoured sub variant across all stakeholders was that introducing specific data quality requirements for sampling points / measurements. The network of National Air Quality Reference Laboratories also strongly favours a revision of data quality objectives.72

K2
Introduce up-to-date data at all sampling points

Policy Measure Env. impact Soc. impact Eco. impact Cost Policy synergy Benefit to cost Included in policy options K2 Introduce up-to-date data at all sampling points 0 (+) 0 0 0 0 0 0 0 0 0

0 Low Sub option III-1c and Policy option IV-1

0 High Policy option III-5

Focus of measure: Introduce a standardized ‘modelling quality objective’ as a quality control mechanism to assess whether a modelling based assessment is fit-for-purpose. Description of measure: Any modelling application used in support of the implementation of the Ambient Air Quality Directives should be of sufficient quality and be fit-for-purpose. This intervention is introducing a standardized Modelling Quality Objective (MQO) that should be met in the validation and QA/QC processes of modelling systems. FAIRMODE has proposed such a MQO which is currently under evaluation for becoming a CEN standard.73 Stakeholder views: A targeted survey provided feedback on ‘to which extent would this specific intervention address the identified (related) shortcomings’.

Summary: Modelling Quality Objective (MQO) would need to be met in the validation and QA/QC processes of modelling systems. FAIRMODE has proposed such a MQO which is currently under evaluation for becoming a CEN standard. High quality modelling

73 Thunis P., Janssen S., Wesseling J.,Piersanti A., Pirovano G., Tarrason L., Martin F., S. Lopez-Aparicio, Bessagnet B., Guevara M., Monteiro A., Clappier A., Pisoni E., Guerreiro C., González Ortiz A., Recommendations for the revision of the ambient air quality directives (AAQDs) regarding modelling applications.

215

applications will contribute to better air quality assessment and planning process. This results in high quality information for the public at large, better source allocation and source identification and eventually better air quality planning. There would be a small administrative burden as some of the modelling systems would have to be upgraded to meet the quality standards.

K4 Revise approach to AQ assessment uncertainty

Policy Measure Env. impact Soc. impact Eco. impact Cost Policy synergy Benefit to cost Included in policy options K4 Revise approach to AQ assessment uncertainty (+) (+) (+) 0 0 0 0 0 0 0 0

0

Medium Sub option III-4a

Focus of measure: Modify the definition of measurement uncertainty by defining it in absolute values and not in percentage values (or a combination of both). Description of measure: Clarification in the definition of measurement uncertainty by establishing these both in absolute values and percentage values, and changes to threshold levels to be achieved. Stakeholder views: A targeted survey provided feedback on ‘to which extent would this specific intervention address the identified (related) shortcomings’.

Summary: Revised monitoring uncertainty and how this is designed, particularly important particularly when air quality standards are low, could improve the quality of measurement data leading to overall improved air quality and reducing health and ecosystem impacts. While it is unlikely to bring significant benefits to air quality management it is an important aspect to clarify. Changes in the calculation for uncertainty may have a negative impact on existing long-established monitoring datasets should it not comply with uncertainty standards. This would negatively impact data quality and overall assessment of pollutant levels for those in non-compliance. Overall, stakeholders saw benefit in combining uncertainty in both absolute and percentage terms.

216

217

2.13 Intervention area L: Additional pollutants
L1 Introduce concept of monitoring at ‘super-sites’

Policy Measure Env. impact Soc. impact Eco. impact Cost Policy synergy Benefit to cost Included in policy options L1 Introduce concept of monitoring at ‘super-sites’ (+) (+) (+) 0 0 0 0 (+) 0 0 0

0

0

High Policy option III-3

Focus of measure: Require monitoring of additional air pollutants at a minimum number of sampling points and with relevant data quality requirements. Description of measure: Requirements for the monitoring of additional pollutants, possibilities for which additional air pollutants should be monitored75, and which include: (1) Ultrafine particles; (2) Ammonia; (3) Fine combustion particles; (4) Oxidative potential; (5) Additional heavy metals; (6) Hydrogen sulphide (H2S) and other reduced sulphur compounds (TRS); (7) Nitro-PAHs; (8) Pesticides. Stakeholder views: A targeted survey provided feedback on ‘to which extent would this specific intervention address the identified (related) shortcomings’.

Summary: Possibilities for additional air pollutants to be monitored include: ultrafine particles, ammonia, fine combustion particles, oxidative potential, additional heavy metals, hydrogen sulphide (H2S) and other reduced sulphur compounds (TRS), nitro-pahs and pesticides. Monitoring of pollutants of emerging concern is essential to advance our understanding of current pollution loads, but also to assess source apportionment and underpin modelling to assess future projected levels. This intervention would facilitate

75 Suggested as such also by the network of National Air Quality Reference Laboratories under their internal Working Group document on suggestions for the Revision of the Ambient Air Quality Directives of December 2021.

219

research on these emerging pollutants and support epidemiological studies of pollutants of most concern to health. Monitoring of air pollution is costly, and even more so for pollutants which are not widely monitored. Administrative burden would be high, and likely to include capacity building to train site operators. For ammonia, monitoring would benefit from coordination with monitoring efforts under the National Emission reduction Commitments Directive76, not least to minimise administrative burden, and a focus on locations where ammonia concentrations could particularly impact ecosystems. Monitoring of pollutants of emerging concern would be essential to setting standards for additional pollutants and the setting up of a priority watch list (links with Policy Area 1 and intervention Ø1 and Policy Area 2 and intervention A4).

L3 Revise list of VOC to monitor

Policy Measure Env. impact Soc. impact Eco. impact Cost Policy synergy Benefit to cost Included in policy options L3 Revise list of VOC to monitor (+) (+) (+) 0 0 0 0 0 0 0 0

0

Low Policy option III-3a

Focus of measure: Expand the list of required and/or recommended volatile organic compounds (VOCs) to measure. Description of measure: Additional VOCs to be monitored should be specified together with monitoring methods, data quality objectives and minimum number and siting requirements and reporting of data. Stakeholder views: A targeted survey provided feedback on ‘to which extent would this specific intervention address the identified (related) shortcomings’.

Summary: Further elaboration of VOC monitoring is necessary to develop scientific knowledge to support emission control, though costs are high (for new analysers to measure

76 Directive (EU) 2016/2284, Article 9

220

more VOCs, and additional resources that may be needed to service and maintain sites, and manage and report data). The network of National Air Quality Reference Laboratories recommends the measurement of appropriate volatile organic compounds suggesting a list of 45 possible substances. Which specifically should be measured would depend on the objective sought.77 However, the merit of monitoring more (or other) VOCs in addition to those regularly monitored is unclear. Further monitoring should be accompanied by data quality and siting specifications with appropriate guidance).

77 As expressed in their internal Working Group document on suggestions for the Revision of the Ambient Air Quality Directives of December 2021.

221

2.14 Intervention area M: Transboundary air pollution
M1 Introduce methodology to assess transboundary

Policy Measure Env. impact Soc. impact Eco. impact Cost Policy synergy Benefit to cost Included in policy options M1 Introduce methodology to assess transboundary (+) (+) (+) (+) 0 0 0 0 0/(+) 0 0/(+)

- (+) High Policy option II-5

Focus of measure: Require the use of an agreed methodology when assessing transboundary air pollution/contributions to local/regional air pollution. Description of measure: Member States face intra-EU transboundary air pollution as well as air pollution from non-EU countries. Currently Article 25 of the 2008/50/EC Directive states that Member States concerned with transboundary air pollution ‘shall’ cooperate to mitigate air pollution (for instance through drawing joint or coordinated air quality plans). The Fitness Check findings highlight that the lack of coordination is likely to affect the understanding of which measures may prove most useful and effective. By offering a common methodology to assess transboundary air pollution, such coordination can be enhanced.
Stakeholder views: A targeted survey provided feedback on ‘to which extent would this specific intervention address the identified (related) shortcomings’.

Summary: This measure aims to facilitate and harmonise the used methodology when assessing transboundary air pollution/contributions to local/regional air pollution. The effectiveness of this intervention to improve air quality is impacted by the willingness of Member States to implement mitigation measures within a joint air quality plan. Implementing this intervention would imply additional costs for Member States who must align their methodology to assess transboundary air pollution. A challenge for implementation is that it may be unclear where the responsibility lies for transboundary air pollution assessment and action. In addition, assessment expertise is needed to conduct the modelling and there is a risk of limited expertise at local level.

222

M2 Revise obligations for transboundary cooperation

Policy Measure Env. impact Soc. impact Eco. impact Cost Policy synergy Benefit to cost Included in policy options M2 Revise obligations for transboundary cooperation (+)
(+)
(+)
(+)
0 0 0

## (+)

## 0

- (+) Medium Policy option II-5

Focus of measure: Require transboundary cooperation and joint action on air quality if assessments of transboundary air pollution/contributions above certain thresholds (to be defined). Description of measure: Under this policy measure, the Ambient Air Quality Directives would require EU Member States at bordering countries to engage in joint action on air quality with neighbouring non-EU countries in cases where air pollution reaches a certain threshold. Member States face intra-EU transboundary air pollution (as well as pollution coming from non-EU countries) which cannot be reduced by one country alone. Article 25 of the 2008/50/EC Directive states that Member States concerned with transboundary pollution ‘shall’ cooperate to mitigate air pollution for instance through drawing joint or coordinated air quality plans. However, such action is currently voluntary and the provision does not specify above which thresholds Member States should seek this cooperation which, in practice, results in lack of cooperation. Stakeholder views: A targeted survey provided feedback on ‘to which extent would this specific intervention address the identified (related) shortcomings’.

Summary: Requiring joint transboundary cooperation above a specific threshold would foster transboundary cooperation and in turn improve air quality in bordering regions, and benefit health and ecosystems in these areas. Implementing this intervention would imply additional costs for competent authorities especially in bordering countries were transboundary air pollution is an issue. Implementation challenges include enforcement (where one Member State cannot enforce action in another), lack of funds at local/regional authority level and acceptability of authorities and industry to implement measures to bring air improvements elsewhere.

223

2.15 Intervention area N: Information in air quality plans
N1 Revise the information in air quality plans

Policy Measure Env. impact Soc. impact Eco. impact Cost Policy synergy Benefit to cost Included in policy options N1 Revise the information in air quality plans ++ + + (+) (+) 0 0 (+) - 0 -

+ High Policy option II-1

+

+++ High but uncertain Policy option I-1 and Sub option I-1a

+

++ High Policy option I-2 and Sub option I-2a

+

- High Policy option I-3 and Sub option I-3a

Focus of measure: EU air quality standards for annual concentrations of PM2.5 Description of measure: The current Ambient Air Quality Directives standards for annual PM2.5 set an annual average limit value of 25 µg/m3. The WHO Air Quality Guidelines are set at 5 µg/m3, alongside higher interim targets. This intervention explores the alignment of the EU long-term standard limit values for PM2.5 with the WHO Air Quality Guidelines updated limit values. Variants of the intervention consider different levels at which the standard can be set below the existing EU standard. A sample of variants has been selected for the modelling in distinct 5 µg/m3 steps. Variants can also change the timeframe over which a standard should be achieved. Stakeholder views: A targeted survey provided feedback on ‘to which extent would this specific intervention address the identified (related) shortcomings’.

Summary: A sample of variants has been selected for the modelling in distinct 5 µg steps: The modelling performed suggests that by 2030, there will be broad compliance with both a 20 and 15 µg/m3 target, with around 400 000 people living in areas of exceedance for the 15 µg/m3 target. More ambitious standards can achieve greater improvements in air quality, with corresponding benefits for health and ecosystems. Administrative burden will also scale

225

with ambition (impacting Member State competent authorities) as the more ambitious the standard, the more new zones will be identified as requiring measures to avoid exceedances. Similarly, mitigation/adjustment costs increase with ambition. The costs of such action are uncertain and depend on the starting point for each one, but these could imply significant change in behaviour at local or national level. As the level of ambition increases, the cost of mitigation/adjustment measures will increase on a non-linear basis. Specific to PM2.5 is the fact that this pollutant may be emitted directly by natural sources. It is also a transboundary pollutant. The extent to which standards can address these issues is uncertain. Stakeholders firmly recognise the value of an annual average standard for PM2.5, which applies as a limit value to all territories in the EU, but opinions vary on what level of ambition is appropriate by when.

O2 Introduce standards for daily PM2.5

Policy Measure Env. impact Soc. impact Eco. impact Cost Policy synergy Benefit to cost Included in policy options O2 Introduce standards for daily PM2.5 : 15 µg/m3
(on 99% days in a year) +++ +++ +++ +++ ++ +/- 0 + — +/- —

+++ High but uncertain Policy option I-1 and Sub option I-1a

+

++ High

+/-

- High Policy option I-3 and Sub option I-3a

Focus of measure: EU air quality standards / thresholds for daily concentrations of PM2.5 Description of measure: The current Ambient Air Quality Directives do not contain a short- term standard for PM2.5. The WHO Air Quality Guidelines set a recommended limit of 15 µg/m3 over a 24-hour period (99th percentile, three to four exceedance days per year), alongside higher interim targets. This intervention explores the value of introducing a new EU short-term limit values for PM2.5 in line with the WHO Air Quality Guidelines. Variants of the intervention consider different levels at which the standard can be set below the existing EU standard. Variants can also change the timeframe over which a standard should be achieved.

226

Stakeholder views: A targeted survey provided feedback on ‘to which extent would this specific intervention address the identified (related) shortcomings’.

Summary: The intervention considers the introduction of a new standard. Variants take the same approach as described for O1. Short-term standards are not modelled explicitly, and hence judgements regarding the balance of costs and benefits is more uncertain. Greater health benefits are typically associated with chronic exposure (in assessment), but where the risk of peaks is quite high and considering this intervention in isolation, the benefits would be much more significant. Administrative burden will also scale with ambition (impacting Member State competent authorities). In addition, short-term compliance measures to tackle peak concentrations specifically may be more disruptive in nature (albeit for a short-time) and carry a higher cost. It appears that there is merit in having a standard to manage peak alongside annual average concentrations – this is underlined by stakeholders and the advice of the WHO, who explore that even a small number of extreme peaks could have a significant impact.

O3 Revise average exposure standards for PM2.5

Policy Measure Env. impact Soc. impact Eco. impact Cost Policy synergy Benefit to cost Included in policy options O3 Revise standards for average exposure for PM2.5 : 5 µg/m3
++ ++ ++ ++ +++ +/- + ++ – + –

++ High but uncertain Policy option I-5

+

+++ High but uncertain Policy option I-1 and Sub option I-1a

+

++ High

+

- High

Policy option I-3 and Sub option I-3a

Focus of measure: EU air quality standards for annual concentrations of PM10 Description of measure: The current air quality standards for annual PM10 under the AAQ Directives set an annual average limit value of 40 µg/m3. The WHO Air Quality Guidelines set an annual average of 15 µg/m3, alongside higher interim targets. This measure explores the alignment of the EU long-term standard limit values for PM10 with the WHO Air Quality Guidelines updated limit values. Variants of the intervention consider different levels at which the standard can be set below the existing EU standard. Variants can also change the timeframe over which a standard should be achieved. Stakeholder views: A targeted survey provided feedback on ‘to which extent would this specific intervention address the identified (related) shortcomings’.

Summary: Modelling shows that this intervention could have a significant positive impact on air quality. The health effects across the variants will scale with the level of ambition, even if health effects are more closely associated with exposure to finer particulate matter (PM2.5). For example the modelling performed suggests that by 2030, there will be broad compliance with a 30 µg/m3 target, with only around 13 000 people living in areas of exceedance in OPT15. Under OPT10, around 2.7 million people remain living in areas exceeding 20 µg/m3,

230

implying a moderate level of effort would be needed at local level to meet this ambition. Under OPT5, 13.7 million remain in areas exceeding the WHO Air Quality Guidelines 15 µg/m3. The mitigation costs of lower standards for PM10 have not been modelled. Many of the measures which mitigate PM2.5 would also mitigate PM10 emissions, hence the measures and costs would be similar. Administrative burden will also scale with ambition (impacting Member State competent authorities). Stakeholders firmly recognise the value of an annual average standard for PM10, which applies as a limit value across all territories of the EU. Furthermore, stakeholders also affirm the additional value of a standard for PM10 alongside PM2.5 and show a general interest for improvement. However, opinion varies on what level of ambition is appropriate and by when it should be achieved. .

P2 Revise standards for daily PM10

Policy Measure Env. impact Soc. impact Eco. impact Cost Policy synergy Benefit to cost Included in policy options P2 Revise standards for daily PM10 : 45 µg/m3
(on 99% days in a year) +++ +++ + + ++ +/- 0 + — +/- —

+++ High but uncertain Policy option I-1 and Sub option I-1a

+/-

++ High

+

++ Low but uncertain Policy option I-5

+

+ Low

+

++ High but uncertain Policy option I-1 and Sub option I-1a

+

- ++ High Policy option I-2 and Sub option I-2a

Q1 Revise standards for annual NO2:
30 µg/m3
+ + + + + +/- + + - + -

- High Policy option I-3 and Sub option I-3a

Focus of measure: EU air quality standards for annual concentrations of NO2 Description of measure: The current Ambient Air Quality Directives standards for annual NO2 set an annual average limit value of 40 µg/m3. The WHO Air Quality Guidelines set the limit at 10 µg/m3, alongside higher interim targets. This measure explores the alignment of the EU long-term standard limit values for NO2 with the WHO Air Quality Guidelines updated limit values. Variants of the measure consider different levels at which the standard can be set below the existing EU standard. Variants can also change the timeframe over which a standard should be achieved. Stakeholder views: A targeted survey provided feedback on ‘to which extent would this specific intervention address the identified (related) shortcomings’.

Summary: The health benefits of action targeting the revision of NO2 concentrations may be smaller (assuming there are no co-benefits by way of particulate or GHG emission reductions). The mitigation costs of lower standards for NO2 have not been modelled, as such contrasting benefits and costs is more uncertain. The modelling does show however a broad alignment with a 20 µg/m3 standard by 2030, and with the WHO Air Quality Guidelines by 2050, with only a small number of people which remain exposed to concentrations above these levels (around four to six million respectively). The additional costs and benefits of these options are both negligible (although in practice a reduction in the standard will help reinforce this delivery). Increasing ambition above the baseline will require the uptake of

234

measures not captured in GAINS, and hence for which the costs are uncertain. However, expert judgement would suggest that costs of localised activity may be more disruptive and imply a higher cost (albeit at a local level). Several challenges for implementation have been identified. Stakeholders firmly recognise the value of an annual-average standard for NO2, applying as a limit value to all territory. Furthermore, stakeholders also show a general interest for improvement but opinion varies on what level of ambition is appropriate and by when it should be achieved. The majority of stakeholders feel alignment with the WHO Air Quality Guidelines would not be appropriate by 2030, but most feel a target in the range from 20- 30 µg/m3 would be achievable, with full alignment to 2050.

Q2 Revise/introduce standards for hourly/daily NO2

Policy Measure Env. impact Soc. impact Eco. impact Cost Policy synergy Benefit to cost Included in policy options Q2 Introduce standards for daily NO2:25 µg/m3
(on 99% days in a year) +++ + ++ + + +/- 0 + — +/- –

++ High but uncertain Policy option I-1 and Sub option I-1a

+/-

- ++ High Policy option I-2 and Sub option I-2a

Q2 Introduce standards for daily NO2: 50 µg/m3
(on 90% days in a year) + + + + + +/- 0 + - +/- -

- High Policy option I-3 and Sub option I-3a

Q2 Maintain standards for hourly NO2: 200 µg/m3
(on 99.99% hours in a year) 0 0 0 0 0 0 0 0 0 0 0

0 0 NA Policy option
I-1, I-2, I-3
and
all sub-options

Focus of measure: EU air quality standards / thresholds for daily concentrations of NO2
Description of measure: The Ambient Air Quality Directives sets a standard for 1-hour NO2 at a limit value of 200 µg/m3. The WHO Air Quality Guidelines does not include a 1-hour limit for NO2, although its 2000 Guidelines78 included a 1-hour limit which is consistent with the EU standard. The WHO Air Quality Guidelines set a standard for 24-hour NO2 at a limit value of 25 µg/m3, alongside higher interim targets. No current EU standard for the 24-hour period exists. Variants of the measure consider different levels at which the standard can be set below the existing EU standard. Variants can also change the timeframe over which a standard should be achieved.

78 WHO (2000), Summary of the WHO guidelines (accessed: 10.06.2022)

235

Stakeholder views: A targeted survey provided feedback on ‘to which extent would this specific intervention address the identified (related) shortcomings’.

Summary: The measure considers both the existing standard (1-hour) and the potential introduction of a new (24-hour) standard. In isolation, there is a strong case for a standard managing NO2 peak concentrations. It appears that there is merit in having a standard to manage peak alongside annual average concentrations – this is underlined by stakeholders and the WHO Air Quality Guidelines, who explore that even a small number of extreme peaks could have a significant impact.79 However, the effectiveness of a peak concentration as a safety net (and indeed its additional value over an annual standard) decreases with the number of allowed exceedance days per year. Short-term standards are not modelled, and hence judgements regarding the balance of costs and benefits is more uncertain. Greater health benefits are typically associated with chronic exposure, but where the risk of peaks is quite high and considering this intervention in isolation, the benefits would be much more significant. The mitigation costs will increase with the level of ambition and will depend on the action taken. Short-term standards have not been modelled, as such the costs of mitigation actions are more uncertain. Expert judgement suggests many of the actions taken to mitigate peak concentrations will be the same as those to tackle annual average concentrations which means costs will be similar. Administrative burden will also scale with ambition (impacting Member State competent authorities).
Q3 Introduce average exposure standards for NO2

Policy Measure Env. impact Soc. impact Eco. impact Cost Policy synergy Benefit to cost Included in policy options Q3 Introduce standards for average exposure for NO2: 10 µg/m3
++ + ++ + + +/- + + – + –

+ Medium but uncertain Policy option I-5

+

+ Medium

Policy option I-5

79 For more information on stakeholders views please see Annex 2.

236

Focus of measure: Introduce average exposure obligations and reduction targets for NO2 Description of measures: This intervention would introduce average exposure concentration obligations and reduction targets for NO2. Variants for this measure are based on different initial concentrations and look at whether the reduction targets should be based on annual or daily exposure, and whether they should be set at a regional or national level. In particular, the following mechanisms are under review:  ECO: Exposure concentration obligation – i.e. ‘based an average level determined on the basis of measurements at urban background locations, reflects population exposure – and to be attained over a given period’;  (N)ERT: (National) exposure reduction target – i.e. ‘a percentage reduction of the average exposure to be attained where possible over a given period’.

The WHO Air Quality Guidelines include targets for NO2 based on concentration values rather than exposure reduction targets. Current provisions in the Ambient Air Quality Directives do not set average exposure obligations or reduction targets for NO2. Stakeholder views: A targeted survey provided feedback on ‘to which extent would this specific intervention address the identified (related) shortcomings’.

Summary: The extent to which this intervention contributes to air quality improvements is partly dependent on the level of ambition. If the average exposure obligation for NO2 is set at the WHO guideline level of 10 µg/m3, the level of ambition may be defined through the design of the exposure reduction target, i.e. the time allowed to reduce the gap between the initial average exposure and the average exposure obligation by a set percentage (e.g. reducing the gap by XX% over YY years). The exposure reduction target required may need to be adjusted in view of specific regional circumstances in some cases. A benefit of setting average exposure targets is that they can complement limit values by (a) targeting background concentrations more specifically and (b) steering further air quality improvements beyond attaining limit values where this is feasible. Benefits to ecosystems will occur as a co-benefit of the measures implemented to attain the reduction targets. Therefore, regardless of the level of ambition, revisions to average exposure targets can facilitate targeted reductions of background levels of NO2, rather than limiting focus on

237

pollution hotspots, and therefore deliver health benefits. This is also important for NO2 as a precursor, including to PM. Costs can be significant depending notably on the level of ambition, arising primarily from measures to attain the reduction targets and administrative burden. There is potential to reduce the administrative burden by taking more coordinated and centralised action.

238

2.19 Intervention area R: EU air quality standards for ozone R1 Introduce standards for peak-season O3

Policy Measure Env. impact Soc. impact Eco. impact Cost Policy synergy Benefit to cost Included in policy options R1 Introduce standards for peak-season O3: 60 µg/m3 +++ + +++ + + +/- 0 +

0 + High but uncertain Policy option I-1 and Sub option I-1a

R1 Introduce standards for peak-season O3:
70 µg/m3 ++ + ++ + + +/- 0 + – - –

0 + High but uncertain

Policy option I-2 and Sub option I-2a

R1 Introduce standards for peak-season O3:
100 µg/m3 + + + + + +/- 0 + - - -

0 + High

Policy option I-3 and Sub option I-3a

Focus of measure: EU air quality standards for peak-season concentrations of O3 Description of measure: This measure explores the revision of the EU long-term standard for O3. The current Ambient Air Quality Directives have a long-term ozone standard aimed at the protection of vegetation. This target value is defined in terms of AOT40 (calculated from 1 hour values), over a May to July averaging period, at 18 000 µg/m3 over five-year average. There is no current EU standard for long-term ozone targeting the protection of human health. The WHO Air Quality Guidelines set a peak season recommendation for average daily maximum 8-hour mean O3 concentrations of 60 µg/m3, in the six consecutive months with the highest six-month running-average O3 concentration. Variants of the measure consider different levels at which the standard can be set below the existing EU standard. Variants can also change the timeframe over which a standard should be achieved, and the type of standard to be set.

Stakeholder views: A targeted survey provided feedback on ‘to which extent would this specific intervention address the identified (related) shortcomings’.

239

Summary: The effectiveness of the intervention will vary with the level of ambition. However, given high levels of existing exceedance, the benefit to air quality is expected to be high. Human health benefits tend to be more linked with exposure to other pollutants and hence can be small. Likewise, ecosystem effects typically comprise a lower proportion of the overall benefit of air quality action, relative to human health effects (albeit this is based on an evidence base which has predominantly focused on the valuation of human health effects, for which by extension is more well explored and understood). The cost of achieving different standards for O3 have not been modelled directly so costs are uncertain. Costs will increase with the level of ambition.
Controlling ozone concentrations is complex and challenging, and is driven in part by control of precursors but also by the meteorological conditions. As such it is questionable whether very ambitious standards for ozone would be feasible in all locations. This is perhaps underlined by the different of opinion amongst stakeholders as to whether limit or target values would be most appropriate. Furthermore, there is currently broad exceedances of both the existing EU target value and the WHO Air Quality Guidelines, as such substantial effort would be required to meet an even stricter target, whereas the benefits of such action (at least in economic impact assessment) often rank below action taken around other pollutants.

R2 Revise standards for 8-hour O3

Policy Measure Env. impact Soc. impact Eco. impact Cost Policy synergy Benefit to cost Included in policy options R2 Revise standards for
8-hour O3: 100 µg/m3
(on 99% days in a year) +++ + ++ + + +/- 0 + — - —

+ High but uncertain Policy option I-1 and Sub option I-1a

R2 Revise standards for 8-hour O3: 120 µg/m3 (on 95% days in a year) ++ + + + + +/- 0 +

- –

0 + High

Policy option I-2 and Sub option I-2a

R2 Revise standards for
8-hour O3: 120 µg/m3
(on 90% days in a year) + + + + + +/- 0 + - - -

0 + High

Policy option I-3 and Sub option I-3a

Focus of measure: EU air quality standards / thresholds for 8-hour concentrations of O3 Description of measure: This measure explores the revision of the EU short-term standard for O3. The current AAQ Directives has a target value for the maximum 8-hour daily mean for ozone of 120 µg/m3 (with 25 permitted exceedances allowed per annum averaged over 3 years). The WHO Air Quality Guidelines set a recommendation for average daily maximum 8-hour mean O3 concentrations of 100 µg/m3 (defined as the 99th percentile). Variants of the intervention consider different levels at which the standard can be set below the existing EU standard. Variants can also change the timeframe over which a standard should be achieved, and the type of standard to be set.

240

Stakeholder views: A targeted survey provided feedback on ‘to which extent would this specific intervention address the identified (related) shortcomings’.

Summary: There remains a clear need for a standard to regulate peak concentrations of ozone. However, controlling ozone concentrations is complex and challenging, and is driven in part by control of precursors but also by the meteorological conditions. The modelling data suggests that there will be broad compliance with the EU standard by 2030, but still substantial non-compliance with the WHO Air Quality Guidelines both under the baseline and the maximum feasible reduction scenario. As such it is questionable whether very ambitious standards for ozone would be feasible in all locations. This is perhaps underlined by the different of opinion amongst stakeholders as to whether limit or target values would be most appropriate. Given the size of existing levels of exceedance, and the challenges in controlling ozone concentrations, the costs of increasing ambition or switching to a limit value might be significant. Human health benefits tend to be more linked with exposure to other pollutants and hence can be small. Likewise, ecosystem effects typically comprise a lower proportion of the overall benefit of air quality action, relative to human health effects (albeit this is based on an evidence base which has predominantly focused on the valuation of human health effects, for which by extension is more well explored and understood).

R3 Introduce average exposure standards for O3

Policy Measure Env. impact Soc. impact Eco. impact Cost Policy synergy Benefit to cost Included in policy options R3 Introduce standards for average exposure for O3 : 60 µg/m3
++ + ++ + + +/- 0 + – - –

+ Low

Policy option I-5

R3 Introduce standards for average exposure for O3 : 70 µg/m3 + + + + + +/- 0 +

+ Low

0

0 + Medium

Policy option
I-1, I-2, I-3
and
all sub-options

Focus of measure: EU air quality standards for annual concentrations of SO₂ Description of measure: This measure explores the revision of the EU long-term standard for SO2. The current Ambient Air Quality Directives set a critical level for the protection of vegetation over the calendar year and winter (1 October to 31 March) of 20 µg/m3, with no margin of tolerance. There is no existing, long-term EU standard for SO2 aimed at the protection of human health. Furthermore, the WHO Air Quality Guidelines do not include a recommendation for long-term exposure to SO2. Variants of the measure consider different levels at which the standard can be set below the existing EU standard. Variants can also change the timeframe over which a standard should be achieved. Stakeholder views: A targeted survey provided feedback on ‘to which extent would this specific intervention address the identified (related) shortcomings’.

Summary: Revisions to this standard were not modelled and therefore the balance of costs and benefits is more uncertain. There has been substantial progress around SO2 emissions and concentrations historically. This may also suggest that a majority of the low-cost actions may have already been captured. Furthermore, the benefits per ton of pollutant abated are smaller than for other pollutants (e.g. PM2.5). The WHO did not include an Air Quality Guidelines recommendation around long-term exposure to SO2 with which an EU standard targeting human health could align. In addition stakeholders provided limited input.

244

S2 Revise standards for daily/hourly SO2

Policy Measure Env. impact Soc. impact Eco. impact Cost Policy synergy Benefit to cost Included in policy options S2 Introduce standards for daily SO2: 40 µg/m3
(on 99% days in a year) + + + + 0 0 0 + - 0 -

0 + Medium

Policy option I-1 and Sub option I-1a

S2 Introduce standards for daily SO2: 50 µg/m3
(on 95% days in a year) + + + + 0 0 0 + - 0 -

0 + Medium

Policy option I-2, I-3 and Sub option I-2a,
I-3a

S2 Maintain standards for hourly SO2: 350 µg/m3
(on 99.98% hours in a year) + + + + 0 0 0 + - 0 -

0 + Medium

Policy option
I-1, I-2, I-3
and
all sub-options

Focus of measure: EU air quality standards / thresholds for daily concentrations of SO2
Description of measure: This measure explores the alignment of the EU short-term limit values for SO2 with the WHO Air Quality Guidelines updated limit values. The current Ambient Air Quality Directives sets a two standards:  A 24-hour limit value of 125 µg/m3 (can be exceeded up to three times per year), which is above the WHO Air Quality Guidelines of 40 µg/m3 (based on 99th percentile);  A 1-hour limit value of 350 µg/m3 (can be exceeded up to 24 times per year). The WHO does not make a recommendation of exposure over a 1-hour averaging period.

The measure also considers the revision of existing and/or the introduction of short-term standards, either alongside or instead of the existing standard. Variants of the measure consider different levels at which the standard can be set below the existing EU standard. Variants can also change the timeframe over which a standard should be achieved. Stakeholder views: A targeted survey provided feedback on ‘to which extent would this specific intervention address the identified (related) shortcomings’.

Summary: This measure considers both: (a) changes to the existing EU limit values and (b) addition to or substitution of the existing EU standard with alternative short-term standards in

245

the WHO Air Quality Guidelines. Revisions to this standard were not modelled and so the balance of costs and benefits is more uncertain. No monitoring data is available over a 10- minute period, which makes it challenging to draw conclusions around the impact of and merit to introducing a new 10-minute standard alongside, or instead of, other short-term standards for SO2. As described for S1, historical progress for SO2 may suggest that low-cost actions have already been captured. Stakeholders propose that the WHO standards could be met with limited additional effort.

246

2.21 Intervention area T: EU air quality standards for carbon monoxide T1 Revise standards for daily/8-hour CO

Policy Measure Env. impact Soc. impact Eco. impact Cost Policy synergy Benefit to cost Included in policy options T1 Introduce standards for daily CO: 4 mg/m3
(on 99% days in a year) + + 0 + 0 0 0 0 - 0 -

0 + High

Policy option I-1 and Sub option I-1a

T1 Introduce standards for daily CO: 4 mg/m3
(on 95% days in a year) + + 0 + 0 0 0 0 - 0 -

0 + High

Policy option I-2 and Sub option I-2a

T1 Introduce standards for daily CO: 7 mg/m3
(on 95% days in a year) + + 0 + 0 0 0 0 - 0 -

0 + High

Policy option I-3 and Sub option I-3a

T1 Maintain standards for
8-hour CO: 10 mg/m3
0 0 0 0 0 0 0 0 0 0 0

0 0 NA

Policy option
I-1, I-2, I-3
and
all sub-options

Focus of measure: EU air quality standards for daily concentrations of CO Description of measure: This measure explores the alignment of the EU short-term limit values for CO with the WHO Air Quality Guidelines updated limit values. The current Ambient Air Quality Directives set a daily 8-hour mean limit value at 10 µg/m3, which corresponds to the standard set by the WHO Air Quality Guidelines. This measure considers going beyond the WHO Air Quality Guidelines for this averaging period. The WHO Air Quality Guidelines also set several other short-term standards, for which an EU standard does not exist:
- The recommended 24-hour WHO standard is set at 4 µg/m3 (measured on the 99th percentile) - The recommended 1-hour WHO standard is set at 35 µg/m3 - The recommended 15 minute WHO standard is set at 100 µg/m3

The 24-hour target was introduced in the 2021 WHO Air Quality Guidelines, with the other three standards being confirmed as remaining valid. The measure also considers the introduction of short-term standards over these averaging periods, either alongside or instead of the existing standard. Variants of the measure consider different levels at which the standard can be set below the existing EU standard. Variants can also change the timeframe over which a standard should be achieved.

247

Stakeholder views: A targeted survey provided feedback on ‘to which extent would this specific intervention address the identified (related) shortcomings’.

Summary: This measure considers both: (a) changes to the existing EU limit value and (b) addition to or substitution of the existing EU standard with alternative short-term standards in the WHO Air Quality Guidelines. From the modelling performed, a certain level of improvement can be made through abatement measures for moderate cost. However, achieving further improvements going beyond the WHO Air Quality Guidelines will require the take up of non-technical or local measures not captured by the modelling, thus the costs are uncertain. Health benefits are more commonly associated with PM2.5, as such the benefits per ton of CO reduction are relatively lower. Stakeholders propose that the existing EU standards can be met with limited additional effort and propose to remain at the existing standard. For the introduction of an additional standard the response to the targeted stakeholder was uncertain.

248

2.22 Intervention area U: EU air quality standards for benzene U1 Revise standards for annual benzene

Policy Measure Env. impact Soc. impact Eco. impact Cost Policy synergy Benefit to cost Included in policy options U1 Revise standards for annual benzene:
1.7 µg/m3
+ + 0 0 0 0 0 0 - 0 -

0 0 Medium

Policy option I-1 and Sub option I-1a

U1 Revise standards for annual benzene:
3.4 µg/m3 + + 0 0 0 0 0 0 - 0 -

0 0 Medium

Policy option I-2 and Sub option I-2a

U1 Maintain standards for annual benzene:
5 µg/m3 0 0 0 0 0 0 0 0 0 0 0

0 0 NA

Policy option I-3 and Sub option I-3a

Focus of measure: Revise EU air quality standards for annual concentrations of benzene Description of measure: This intervention explores the alignment of the EU long-term standard limit values for benzene with the WHO Air Quality Guidelines, which for benzene were contained in the 2000 WHO Air Quality Guidelines. The current Ambient Air Quality Directives set an annual average limit value for benzene of 5 µg/m3. The WHO standard is set at 1.7 µg/m3. Variants of the measure consider different levels at which the standard can be set below the existing EU standard. Variants can also change the timeframe over which a standard should be achieved.

Stakeholder views: A targeted survey provided feedback on ‘to which extent would this specific intervention address the identified (related) shortcomings’.

Summary: There is broad compliance with the existing standard in 2019 and low exceedances relative to the WHO Air Quality Guidelines, not accounting for further improvements in the baseline. The negative impact of benzene is however also lower in relation to other pollutants.

249

2.23 Intervention area V: EU air quality standards for benzo(a)pyrene V1 Revise standards for annual benzo(a)pyrene

Policy Measure Env. impact Soc. impact Eco. impact Cost Policy synergy Benefit to cost Included in policy options V1 Revise standards for annual benzo(a)pyrene:
0.12 ng/m3
++ ++ 0 + + +/- +/- + - +/- -

+ Medium

+/-

+ Medium

+/-

+ Low but uncertain

Even if all effort is made, the related targets cannot be fully achieved everywhere (due to physical geography constraints). But at locations where achieved, they bring major health benefits. High but uncertain + I-1a: by 2050 ++ ++ ++

See above. High but uncertain

Policy option I-2 Closer alignment with WHO recommendations ++ ++ ++ – Current baseline policies bring most regions close to target. Achieving this target has considerable health benefits and social co-benefits – medium effort needed. High + I-2a: by 2050 + + + - Target would be achievable with little extra effort. High

Policy option I-3 Partial alignment with WHO recommendations + + + - Current baseline policies will achieve this level in almost all of the EU. Thus setting targets at this level offers only limited added benefit (but where it triggers additional action this is of high benefit). High + I-3a: by 2050 0 0 0 0 Likely does not require additional policy action. NA

Policy option I-4: Additional air pollutants Policy option Consequences / Impacts Assessment and key considerations Benefit to cost Env. Soc. Eco. Cost Policy option I-4 Additional air pollutants + +/- +/- – May have benefits, but to date no basis in WHO recommendations to set such air quality standards. Priority should be establishing a monitoring network for these pollutants (see III-1). Low but uncertain

Policy Measure Env. impact Soc. impact Eco. Impact Cost Policy synergy Benefit to cost Included in policy options Ø1 Introduce standards for additional air pollutants ++ + + + + +/- +/- + – +/- –

+ Low but uncertain

Can build on existing concept and monitoring, but at more appropriate regional resolution, to help assure continuous decrease in background PM2.5. High + I-5a: PM10 + + +/-

Low added value, if PM2.5 is already covered. Low + I-5b: NO2 + + + – Extra burden, NO2 focus better be ‘hotspots’.
Medium + I-5c: O3 + +/- +/- – Uncertain if O3 metric can trigger effective action. Uncertain

Policy Measure Env. impact Soc. impact Eco. impact Cost Policy synergy Benefit to cost Included in policy options B3 Revise definition of average exposure standards (+) (+) (+) (+) (+) 0 0 (+) 0 0 0

(-) (+) High Policy option I-5

257

Policy option I-6: Review air quality standards Policy option Consequences / Impacts Assessment and key considerations Benefit to cost Env. Soc. Eco. Cost Policy option I-6 Review air quality standards (Measures A1,A3) ++ + 0 - Regular review will ensure scientific evidence base of EU policy making, but should be spaced to allow for sufficient scientific progress and regulatory certainty.
High + I-6a: Measure A2 + 0 0 - Little extra value compared to main option. Low + I-6b: Measure A4 + 0 0 – High (admin) burden for uncertain added value. Low

Policy Measure Env. impact Soc. impact Eco. impact Cost Policy synergy Benefit to cost Included in policy options A1 Introduce review triggered by scientific progress ++ (+) (+) + (+) (+) 0

0 0 0 0

(-) + High

Policy option I-6
A3 Introduce option to notify stricter standards (+) (+) (+) (+) (+) 0 0 0 0 0 0

(-) 0 High

Policy option I-6 +

Policy Measure Env. impact Soc. impact Eco. impact Cost Policy synergy Benefit to cost Included in policy options A2 Introduce review triggered by technical progress (+) (+) (+) (+) 0 0 0 0 0 0 0

(-) (+) Low

Sub-option I-6a
+

Policy Measure Env. impact Soc. impact Eco. impact Cost Policy synergy Benefit to cost Included in policy options A4 Introduce a list of priority pollutants
(+) (+) (+) (+) 0 0 0 0 0 0 0

0 Low

This policy option will update the means by which air quality plans are developed. Costs to change existing approach compensated or even reduced by more effective air quality plans and measures. Medium + II-1a: Measure D2 0 0 0

Added value doubtful, subsidiarity considerations. Low

Policy Measure Env. impact Soc. impact Eco. impact Cost Policy synergy Benefit to cost Included in policy options B4 Introduce guidance on addressing exceedances (+) (+) (+) (+) 0 0 0 0 0 0 0

(-) (+) Medium Policy option II-1 C1 Revise obligations triggered by exceedances (+) (+) (+) (+) 0 (+) 0 (+) 0 0 0

(+) Medium / High Policy option II-1 Policy option II-4 C3 Revise short-term action plans & air quality plans (+) (+) (+) 0 0 (+) 0 (+) 0 0 0

0 Medium Policy option II-1

258

D1 Revise requirements to involve stakeholders (+) (+) (+) 0 0 0 0 0 0/- 0 0/-

0 High Policy option II-1 N1 Revise the information in air quality plans ++ + + (+) (+) 0 0 (+) - 0 -

+ High Policy option II-1 +

0 Low Sub-option II-1a Policy option II-2: Additional limit values Policy option Consequences / Impacts Assessment and key considerations Benefit to cost Env. Soc. Eco. Cost Policy option II-2 Additional limit values (Measures B1,B5) ++ + +/-

Fitness Check indicates that ‘limit values’ have been more effective than other types of air quality standards. For some pollutants (notably O3), however the concept is unlikely to have benefits. High

Policy Measure Env. impact Soc. impact Eco. impact Cost Policy synergy Benefit to cost Included in policy options B1 Introduce additional short- term standards
+ 0 0 0 0 0 0 0 0 0 0

(-) 0 High

Policy option II-2 B5 Introduce limit values for additional air pollutants
++ + + + + 0 0 + - 0 -

(-) 0 Medium Policy option II-2 Policy option II-3: Implementation timelines Policy option Consequences / Impacts Assessment and key considerations Benefit to cost Env. Soc. Eco. Cost II-3 Implementation timelines & short-term action plans (Measures B2,C2,C4,C5) + + +/- – The key added value would be to ensure regular updates of air quality plans. Alert thresholds for particulate matter would address additional health concerns, but likely at a cost. Medium

Policy Measure Env. impact Soc. impact Eco. impact Cost Policy synergy Benefit to cost Included in policy options B2 Introduce additional alert/information thresholds + + (+) (+) + 0 0 0 0 0 0

(-) (+) Medium Policy option II-3 C2 Revise/clarify definition of ‘as short as possible’ (+) (+) (+) (+) 0 (+) 0 (+) 0 0 0

(+) Medium Policy option II-3 C4 Introduce additional short- term action plans + + + (+) + (+) (-) (+) 0/- (-) 0/-

(+) Medium Policy option II-3 C5 Introduce requirement to update air quality plans (+) (+) (+) (+) 0 (+) 0

(+)

## 0

(+) Medium Policy option II-3

Subsidiarity to be considered, unclear how. Uncertain

Policy Measure Env. impact Soc. impact Eco. impact Cost Policy synergy Benefit to cost Included in policy options C1 Revise obligations triggered by exceedances (+) (+) (+) (+) 0 (+) 0 (+) 0 0 0

(+) Medium / High Policy option II-1 Policy option II-4 E1 Introduce minimum levels for financial penalties
(+)
(+)
(+)
(+)
0
(+)
0 (+) (-)
0

(-)

(-)

(+) Medium / High Policy option II-4 E2 Introduce right to health damage compensation (+)
(+) (+)
(+) 0 (+) 0 (+) - 0 -

0

(+) Medium Policy option II-4 E4 Introduce access to justice provision (+)
(+)
(+)
(+)
0 (+) 0

(+) (-)
0 (-)

0 0 High Policy option II-4

Policy Measure Env. impact Soc. impact Eco. impact Cost Policy synergy Benefit to cost Included in policy options E3 Introduce a fund to be fed by penalties paid 0/(+) 0/(+) 0/(+) 0/(+) 0 (+) 0 0 0/(+) 0 0/+

(-) 0/(+) Low Sub-option II-4a Policy option II-5: Transboundary air pollution Policy option Consequences / Impacts Assessment and key considerations Benefit to cost Env. Soc. Eco. Cost Policy option II-5 Transboundary air pollution (Measures M1,M2)

- 0 +/-

- Transboundary air pollution is already the focus of the NEC Directive. Further cooperation is desirable but difficult to enforce. Additional guidance helpful. Medium

Policy Measure Env. impact Soc. impact Eco. impact Cost Policy synergy Benefit to cost Included in policy options M1 Introduce methodology to assess transboundary (+) (+) (+) (+) 0 0 0 0 0/(+) 0 0/(+)

(+) High Policy option II-5 M2 Revise obligations for transboundary cooperation (+) (+) (+) (+) 0 0 0

0

- (+) Medium Policy option II-5

260

3.3 Policy options to address air quality monitoring / assessment shortcomings
Policy option III-1: Air quality assessments Policy option Consequences / Impacts Assessment and key considerations Benefit to cost Env. Soc. Eco. Cost Policy option III-1
Air quality assessments (Measures G1, G2, H1, H2, L1) + + + – Will significantly improve air quality monitoring and assessment, allowing for more targeted air quality measures, and make better use of avail. methods. Costs related to the expansion of the monitoring network and adding ‘super-sites’. Medium + III-1a: Measure G3 0 0 0 0 Minor admin. simplification only, but at (low) cost. Low + III-1b: Measure H3 0 0 0 - Minor admin. simplification only, but at (low) cost. Low + III-1c: Measure K2 + 0 0 -
Will improve data, but at potentially high cost. Low + III-1d: Measure J3 + + + – Will allow more targeted air quality management. Medium

Policy Measure Env. impact Soc. impact Eco. impact Cost Policy synergy Benefit to cost Included in policy options G1 Revise rules related to indicative sampling points (+) (+) (+) 0 0 (+) 0 (+) 0 0 0

0 High Policy option III-1 G2 Introduce requirements for AQ modelling (+) (+) (+) 0 (+) (+) 0 0 0 0 0

0

Medium / High Policy option III-1 and Policy option III-5 H1 Revise minimum number of sampling points + + + 0 0 0 0 +/- 0 0 0

0

Medium Policy option III-1 H2 Simplify combined PM10/PM2.5 monitoring (+) (+) 0 0 0 0 0 (+) 0 0 0

0 High Policy option III-1 L1 Introduce concept of monitoring at ‘super-sites’ (+) (+) (+) 0 0 0 0 (+) 0 0 0

0

Medium Policy option III-1 and Policy option III-3 +

Policy Measure Env. impact Soc. impact Eco. impact Cost Policy synergy Benefit to cost Included in policy options G3 Revise rules for regular review of AQ assessment 0 0 0 0 0 0 0 0 0 0 0

(-) 0 Low Sub option III-1a +

Policy Measure Env. impact Soc. impact Eco. impact Cost Policy synergy Benefit to cost Included in policy options H3 Simplify the definitions of sampling points types 0 0 0 0 0 0 0 0 0 0 0

0

Low Sub option III-1b +

Policy Measure Env. impact Soc. impact Eco. impact Cost Policy synergy Benefit to cost Included in policy options K2 Introduce up-to-date data at all sampling points 0 (+) 0 0 0 0 0 0 0 0 0

0 Low Sub option III-1c and Policy option IV-1 +

Policy Measure Env. impact Soc. impact Eco. impact Cost Policy synergy Benefit to cost Included in policy options J3 Introduce obligation for spatial representativeness (+) (+) (+) 0 (+) 0 0 (+) 0 0 0

0

Medium Sub option III-3d

261

Policy option III-2: Monitoring continuity Policy option Consequences / Impacts Assessment and key considerations Benefit to cost Env. Soc. Eco. Cost Policy option III-2 Monitoring continuity (Measures I1, I3) + + 0 - Will significantly improve air quality monitoring and assessment, allowing for more targeted air quality measures.
Medium + III-2a: Measure I2 + 0 0 - Minor admin. simplification only, at (low) cost. Low

Policy Measure Env. impact Soc. impact Eco. impact Cost Policy synergy Benefit to cost Included in policy options I1 Introduce obligations to maintain sampling points (+) (+) (+) 0 (+) 0 0 0 0 0 0

0 0 High Policy option III-2 I3 Introduce a protocol for relocated sampling points (+) 0 0 0 (+) 0 0 0 0 0 0

- 0

Medium Policy option III-2 +

Policy Measure Env. impact Soc. impact Eco. impact Cost Policy synergy Benefit to cost Included in policy options I2 Introduce obligations to monitor long-term trends (+) (+) (+) 0 0 0 0 0 0 0 0

(-)

0 Low Sub-option III-2a Policy option III-3: Additional sampling points Policy option Consequences / Impacts Assessment and key considerations Benefit to cost Env. Soc. Eco. Cost Policy option III-3 Additional sampling points (Measures L1, L2) ++ 0 + –/— Clarifies current levels of these air pollutants, as a requisite for verifying health effects and need for taking action. Costs related to the expansion of the monitoring network and adding ‘super-sites’. Medium + III-3a: Measure L3 + 0 0 –/— Can only be a gross list of VOC – needed in law? Low

Policy Measure Env. impact Soc. impact Eco. impact Cost Policy synergy Benefit to cost Included in policy options L1 Introduce concept of monitoring at ‘super-sites’ (+) (+) (+) 0 0 0 0 (+) 0 0 0

0

0

High Policy option III-3 +

Policy Measure Env. impact Soc. impact Eco. impact Cost Policy synergy Benefit to cost Included in policy options L3 Revise list of VOC to monitor (+) (+) (+) 0 0 0 0 0 0 0 0

0

Low Policy option III-3a

262

Policy option III-4: Monitoring data quality Policy option Consequences / Impacts Assessment and key considerations Benefit to cost Env. Soc. Eco. Cost Policy option III-4 Monitoring data quality (Measures J1, J2, K1) + + 0 - Additional clarity will enhance reliability and comparability of air quality data – but may also result in significant cost to update existing air quality monitoring and assessment networks. Medium + III-4a: Measure K4 + 0 0 - Will increase confidence in air quality further. Medium

Policy Measure Env. impact Soc. impact Eco. impact Cost Policy synergy Benefit to cost Included in policy options J1 Revise macro-scale siting of sampling points (+) (+) (+) 0 (+) 0 0 0 0 0 0

0

0

Medium Policy option III-4 K1 Revise AQ monitoring data quality objectives (+) (+) (+) 0 0 0 0 0 0 0 0

0

Medium Policy option III-4 +

Policy Measure Env. impact Soc. impact Eco. impact Cost Policy synergy Benefit to cost Included in policy options K4 Revise approach to AQ assessment uncertainty (+) (+) (+) 0 0 0 0 0 0 0 0

0

This policy option is a prerequisite to an effective implementation of policy option III-1. Important for robust data, but little other direct consequences. Medium

Policy Measure Env. impact Soc. impact Eco. impact Cost Policy synergy Benefit to cost Included in policy options G2 Introduce requirements for AQ modelling (+) (+) (+) 0 (+) (+) 0 0 0 0 0

0

Medium / High Policy option III-1 and Policy option III-5 K3 Introduce AQ modelling data quality objectives (+) (+) (+) 0 0 0 0 0 0 0 0

- 0 High Policy option III-5

263

3.4 Policy options to address air quality information shortcomings
Policy option IV-1: Up-to-date air quality data Policy option Consequences / Impacts Assessment and key considerations Benefit to cost Env. Soc. Eco. Cost Policy option IV-1
Up-to-date air quality data (Measures F1, K2) + + 0 -/– Up-to-date data provision will allow more for additional societal responsiveness to pollution peaks. Related costs will vary, and include a punctual expansion of the monitoring network.
Medium

Policy Measure Env. impact Soc. impact Eco. impact Cost Policy synergy Benefit to cost Included in policy options F1 Revise provisions related to up-to-date data 0 (+) 0 0 (+) 0 0 0 0 0 0

0 Medium / High Policy option IV-1 K2 Introduce up-to-date data at all sampling points 0 (+) 0 0 0 0 0 0 0 0 0

0 Low Sub option III-1c and Policy option IV-1 Policy option IV-2: Health related air quality data Policy option Consequences / Impacts Assessment and key considerations Benefit to cost Env. Soc. Eco. Cost Policy option IV-2 Health related air quality data (Measure F2) + + 0

Potentially impactful measure, will require closer interaction between health practitioners and policy makers to inform a wider public (and vulnerable populations) better. Likely significant initial costs.
Medium + IV-2a: Measure F3 0 0 0 – No added value of specifying channels in law. Low

Policy Measure Env. impact Soc. impact Eco. impact Cost Policy synergy Benefit to cost Included in policy options F2 Introduce requirement to provide AQ health data 0 (+) 0 0 (+) 0 0 0 0 0 0

- 0 Medium Policy option IV-2

Policy Measure Env. impact Soc. impact Eco. impact Cost Policy synergy Benefit to cost Included in policy options F3 Introduce use of specific communication channels 0 0 0 0 (+/-) 0 0 0 0 0 0

0 Low Sub-option IV-2a Policy option IV-3: Harmonised air quality indices Policy option Consequences / Impacts Assessment and key considerations Benefit to cost Env. Soc. Eco. Cost Policy option IV-3 Harmonised air quality indices (Measure F4) + + 0

Harmonisation of air quality data saves costs for developing and updating separate indices. Provides clarity for citizens across the EU. Medium

Policy Measure Env. impact Soc. impact Eco. impact Cost Policy synergy Benefit to cost Included in policy options F4 Introduce requirements for harmonised AQ index 0 (+) 0 0 (+) 0 0 0 0 0 0

0 Medium Policy option IV-3

264

- HOW DO THE OPTIONS COMPARE? Policy option Consequences / Impacts Benefit
to cost Synergies, complementarities and trade-offs with other options and/or sub-options

Env. Soc. Eco. Cost

I-1
Full alignment with WHO recommendations +++ +++ +++ — High but
uncertain Consider a variant of sub-option I-1a (i.e. more ambitious objectives in a post -2030 perspective). Not the preferred option for 2030 compared to I-2. I-2 Closer alignment with WHO recommendations ++ ++ ++ – High Preferred option for air quality objectives for 2030. Effectiveness depends on policy option II-2 - [Discard sub-option I-2a]
I-3 Partial alignment with WHO recommendations + + + - High Not the preferred option for 2030 compared to I-2. - [Discard sub-option I-3a]
I-4 Additional air pollutants 0/+ +/- +/- - Low but uncertain

Efficiency is higher discarding this policy option, but retaining monitoring of additional air pollutants via policy option III-4. I-5 Average exposure reduction
++ + + - High Add complementary sub-option I-5b (NO2),
and consider a variant of sub-option I-5c (O3) - [Discard sub-option I-51] I-6 Review air quality standards ++ + 0 - High - [Discard sub-options I-6a, I-6b]
II-1
Responses to exceedances ++ + +/- – Medium - [Discard sub-option II-1a] II-2 Additional limit values ++ + +/- - High Proportionality of this option also depends on whether policy option I-1, I-2, or I-3 is preferred. II-3 Implementation timelines & revised short-term action plans + + +/- – Medium

II-4 Enforcement tools ++ + +/- 0/- High Consider also complementary sub-option II-4a.
II-5 Transboundary air pollution + 0 +/- - Medium Effectiveness and efficiency depend on refined approaches to policy options II-4 and III-1 III-1
Air quality assessments + + + – Medium Add complementary sub-option III-1d to enhance effectiveness of this option. - [Discard sub-options III-1a, III-1b, III-1c]
III-2 Monitoring continuity + + 0 - Medium - [Discard: Sub-option III-2a] III-3 Additional sampling points ++ 0 + –/— Medium - [Discard: Sub-option III-3a] III-4 Monitoring data quality + + 0 – Medium Add complementary sub-option III-4a to enhance effectiveness of this option.
Efficiency of monitoring requirement for additional pollutants is higher than standards (option I-4). III-5 Modelling data quality + + 0 -/– Medium Complements option III-1. IV-1
Up-to-date air quality data + + 0 -/– Medium Efficiency of this option depends on whether and how options III-1 and III-4 are defined. IV-2 Health related air quality data + + 0 - Medium - [Discard: Sub-option IV-2a] IV-3 Harmonised air quality indices + + 0 - Medium Effectiveness depends on options IV-1 and IV-2 being implemented also.